CpqCisse Event Log Codes

The purpose of this document is to list the Event ID’s of every possible event logged by the device driver cpqcisse.sys to the Windows NT event log.  Cpqcisse.sys works in conjunction with cpqcissm.sys which is the miniport driver for the array controller.  Cpqcisse.sys supports the below storage products:

- Smart Array 5300 Controller

- Smart Array 5i

- Smart Array 532 Controller

- Smart Array 5312 Controller

- Smart Array 641 Controller

- Smart Array 642 Controller

- Smart Array 6400 Controller

- Smart Array 6400 U320 EM Controller

- Smart Array Cluster Storage

Hewlett-Packard Company shall not be liable for technical or editorial errors or omissions contained herein. The information in this document is provided “as is” without warranty of any kind and is subject to change without notice. The warranties for HP products are set forth in the express limited warranty statements accompanying such products. Nothing herein should be construed as constituting an additional warranty.

| Event ID: | 24577 |
| --- | --- |
| Type: | Information |
| Message: | No Events reported for the [Array controller location] |

| Event ID: | 24578 |
| --- | --- |
| Type: | Information |
| Message: | The Event Notifier has been disabled for the [Array controller location] |

| Event ID: | 24579 |
| --- | --- |
| Type: | Information |
| Message: | Event Queue Overflow on the [Array controller location]. At least one event notification has been lost. This controller will only queue up to 100 events. |

| Event ID: | 24580 |
| --- | --- |
| Type: | Information |
| Message: | Physical Drive [Action]: SCSI Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24581 |
| --- | --- |
| Type: | Information |
| Message: | Physical Drive [Action]: SCSI ID [Number] of [Array controller location]. |

| Event ID: | 24582 |
| --- | --- |
| Type: | Information |
| Message: | Redundant Power Supply [Action]: SCSI Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24583 |
| --- | --- |
| Type: | Information |
| Message: | External Fan [Action]: SCSI Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24584 |
| --- | --- |
| Type: | Information |
| Message: | UPS [Action]: SCSI Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24585 |
| --- | --- |
| Type: | Information |
| Message: | [Array controller location] has detected more than 1 cable with active drives attached. See next event (UP ARROW) for Controller message. |

| Event ID: | 24586 |
| --- | --- |
| Type: | Warning |
| Message: | Too Many runtime ECC errors on [Array controller location]. Controller restarted without SIMM. See next event (UP ARROW) for Controller message. |

| Event ID: | 24587 |
| --- | --- |
| Type: | Warning |
| Message: | Fan [Number] of [Location]ternal storage device on [Location] Port [Number] of [Array controller location], has reported a fault. |

| Event ID: | 24588 |
| --- | --- |
| Type: | Warning |
| Message: | Sensor [Number] of [Location]ternal storage device on [Location] Port [Number] of [Array controller location], is reporting the internal temperature has exceed the preset limit. The controller may attempt to shut down power to the storage device and/or spin down the installed disk drives. |

| Event ID: | 24589 |
| --- | --- |
| Type: | Warning |
| Message: | Sensor [Number] of [Location]ternal storage device on [Location] Port [Number] of [Array controller location], is reporting the temperature is nearing the preset temperature limit. The controller may enact precautionary measures to prevent data loss should the temperature reach the operating limit. |

| Event ID: | 24590 |
| --- | --- |
| Type: | Information |
| Message: | Sensor [Number] of [Location]ternal storage device on [Location] Port [Number] of [Array controller location], is reporting the correction of a previously existing temperature condition. All of the temperature sensors in the [Location]ternal storage device are now reporting acceptable temperature levels. |

| Event ID: | 24591 |
| --- | --- |
| Type: | Warning |
| Message: | Redundant Power supply [Number] of External Storage device on Port [Number] of [Array controller location] has reported a fault. |

| Event ID: | 24592 |
| --- | --- |
| Type: | Information |
| Message: | Redundant Power supply [Number] of External Storage device on Port [Number] of [Array controller location] is now operational, or has been removed from the chassis. |

| Event ID: | 24593 |
| --- | --- |
| Type: | Warning |
| Message: | The chassis door of External Storage System on Port [Number] of [Array controller location] is open. The controller may attempt to shut down the power to the storage device and/or spin down the installed disk drives. |

| Event ID: | 24594 |
| --- | --- |
| Type: | Warning |
| Message: | Power supply [Number] of [Location]ternal Storage Device on [Location]Port [Number] of [Array controller location], has reported an AC Power failure. |

| Event ID: | 24595 |
| --- | --- |
| Type: | Warning |
| Message: | Power supply [Number] of [Location]ternal storage Device on [Location]Port [Number] of [Array controller location], has reported a Battery Low condition. |

| Event ID: | 24596 |
| --- | --- |
| Type: | Error |
| Message: | Physical Drive [Number] of [Array controller location], has failed. Failure Code: [Controller specific failure code]. |

| Event ID: | 24597 |
| --- | --- |
| Type: | Error |
| Message: | Physical Drive on SCSI Port [Number], ID [Number] of [Array controller location], has failed. Failure Code: [Controller specific failure code]. |

| Event ID: | 24598 |
| --- | --- |
| Type: | Information |
| Message: | Logical Drive [Number] of [Array controller location] has changed from status code [Number] to status code [Number]. Status Codes: 0 = OK 1 = FAILED 2 = NOT CONFIGURED 3 = INTERIM RECOVERY MODE 4 = READY FOR RECOVERY 5 = RECOVERING 6 = WRONG PHYSICAL DRIVE REPLACED 7 = PHYSICAL DRIVE NOT PROPERLY CONNECTED 8 = HARDWARE IS OVERHEATING 9 = HARDWARE HAS OVERHEATED 10 = EXPANDING 11 = NOT YET AVAILABLE 12 = QUEUED FOR EXPANSION 13 = UNKNOWN |

| Event ID: | 24599 |
| --- | --- |
| Type: | Warning |
| Message: | Logical Drive [Number] of [Array controller location] has changed from status code [Number] to status code [Number]. Status Codes: 0 = OK 1 = FAILED 2 = NOT CONFIGURED 3 = INTERIM RECOVERY MODE 4 = READY FOR RECOVERY 5 = RECOVERING 6 = WRONG PHYSICAL DRIVE REPLACED 7 = PHYSICAL DRIVE NOT PROPERLY CONNECTED 8 = HARDWARE IS OVERHEATING 9 = HARDWARE HAS OVERHEATED 10 = EXPANDING 11 = NOT YET AVAILABLE 12 = QUEUED FOR EXPANSION 13 = UNKNOWN |

| Event ID: | 24600 |
| --- | --- |
| Type: | Error |
| Message: | Logical Drive [Number] of [Array controller location] has changed from status code [Number] to status code [Number]. Status Codes: 0 = OK 1 = FAILED 2 = NOT CONFIGURED 3 = INTERIM RECOVERY MODE 4 = READY FOR RECOVERY 5 = RECOVERING 6 = WRONG PHYSICAL DRIVE REPLACED 7 = PHYSICAL DRIVE NOT PROPERLY CONNECTED 8 = HARDWARE IS OVERHEATING 9 = HARDWARE HAS OVERHEATED 10 = EXPANDING 11 = NOT YET AVAILABLE 12 = QUEUED FOR EXPANSION 13 = UNKNOWN |

| Event ID: | 24601 |
| --- | --- |
| Type: | Information |
| Message: | Logical drive [Number] configured on [Array controller location], is in a FAILED state but has had one or more drive replacements and is ready to go to OK. However, this will not happen until an Accept Media Exchange command is issued to the logical drive. |

| Event ID: | 24602 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive [Number] due to an unrecoverable read error. The physical drive reporting the error is [Number]. |

| Event ID: | 24603 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive with SCSI ID [Number] on SCSI Port [Number] due to an unrecoverable read error. The physical drive reporting the error is SCSI ID [Number] on SCSI Port [Number]. |

| Event ID: | 24604 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive [Number] due to an unrecoverable write error. The physical drive reporting the error is [Number]. |

| Event ID: | 24605 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive with SCSI ID [Number] on SCSI Port [Number] due to an unrecoverable write error. The physical drive reporting the error is SCSI ID [Number] on SCSI Port [Number]. |

| Event ID: | 24606 |
| --- | --- |
| Type: | Error |
| Message: | Logical drive [Number] configured on [Array controller location], has returned a fatal error on a read/write request on the volume. Logical block address [Number], block count [Number], command [Number], drive bus [Number] and Drive Id [Number]. |

| Event ID: | 24607 |
| --- | --- |
| Type: | Warning |
| Message: | The event notification information received from [Array controller location] was of unknown or unrecognized class. Controller message : [Text string]. |

| Event ID: | 24608 |
| --- | --- |
| Type: | Information |
| Message: | External Array Controller attached to [Array controller location] is now active. |

| Event ID: | 24609 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is now redundant. |

| Event ID: | 24610 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because an Array Controller is missing. |

| Event ID: | 24611 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because the Array Controllers are different models. The standby Array Controller has been marked as failed. |

| Event ID: | 24612 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because the inter-controller link has failed. |

| Event ID: | 24613 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because the Array Controllers have different levels of firmware. The standby Array Controller has been marked as failed. |

| Event ID: | 24614 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because both Array Controllers cannot see all the physical drives. The standby Array Controller has been marked as failed. |

| Event ID: | 24615 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because one or more drives has been determined not to be able to support redundant controller operation. The standby Array Controller has been marked as failed. |

| Event ID: | 24616 |
| --- | --- |
| Type: | Information |
| Message: | Redundant Controller [Hot Plug Action] on Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24617 |
| --- | --- |
| Type: | Information |
| Message: | Fan [Number] of [Location]ternal Storage device on [Location] Port [Number] of [Array controller location], is reporting a degraded condition. |

| Event ID: | 24618 |
| --- | --- |
| Type: | Information |
| Message: | Fan [Number] of [Location]ternal storage device on [Location]Port [Number] of [Array controller location], is now operational. |

| Event ID: | 24619 |
| --- | --- |
| Type: | Information |
| Message: | The chassis door of External Storage device [Number] of [Array controller location], is closed. |

| Event ID: | 24623 |
| --- | --- |
| Type: | Information |
| Message: | Tape Drive [Action] : SCSI Port [Number], ID [Number], of [Array controller location]. |

| Event ID: | 24624 |
| --- | --- |
| Type: | Information |
| Message: | Tape Drive [Action] : SCSI ID [Number], of [Array controller location]. |

| Event ID: | 24625 |
| --- | --- |
| Type: | Information |
| Message: | Physical Drive [Action], on Box [Location] and Bay [Location] of [Array controller location]. |

| Event ID: | 24626 |
| --- | --- |
| Type: | Information |
| Message: | Tape Drive [Action], on Box [Location] and Bay [Location] of [Array controller location]. |

| Event ID: | 24627 |
| --- | --- |
| Type: | Information |
| Message: | Redundant Power Supply [Action], on Box [Number], Port [Number] and ID [Number] of [Array controller location]. |

| Event ID: | 24628 (SA Cluster Storage) |
| --- | --- |
| Type: | Information |
| Message: | External Fan [Action], in Box [Number], Port [Number] and ID [Number] of [Array controller location] |

| Event ID: | 24632 |
| --- | --- |
| Type: | Information |
| Message: | The External Storage System of [Array controller location], has reported that the Cache has been temporarily disabled. |

| Event ID: | 24633 |
| --- | --- |
| Type: | Information |
| Message: | The External Storage System of [Array controller location], has reported that the Cache has been re-enabled. |

| Event ID: | 24634 |
| --- | --- |
| Type: | Information |
| Message: | The External Storage System of [Array controller location], has reported that the Cache batteries have been removed. |

| Event ID: | 24638 |
| --- | --- |
| Type: | Warning |
| Message: | Redundant Power supply [Number] of External Storage device on Box [Number], Port [Number] of [Array controller location] has reported a fault. |

| Event ID: | 24639 |
| --- | --- |
| Type: | Information |
| Message: | Redundant Power supply [Number] of External Storage device on Box [Number], Port [Number] of [Array controller location] is now operational, or has been removed from the chassis. |

| Event ID: | 24640 |
| --- | --- |
| Type: | Information |
| Message: | UPS [Action]: Box [Number], Port [Number], ID [Number] of [Array controller location]. |

| Event ID: | 24641 |
| --- | --- |
| Type: | Warning |
| Message: | The chassis door of External Storage System on Box [Number], Port [Number] of [Array controller location] is open. The controller may attempt to shut down the power to the storage device and/or spin down the installed disk drives. |

| Event ID: | 24644 |
| --- | --- |
| Type: | Error |
| Message: | Physical Drive in Box [Number], Bay [Number] of [Array controller location] has failed. Failure Code: [Number] |

| Event ID: | 24645 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive in Box [Number], Bay [Number] due to an unrecoverable read error. The physical drive reporting the error is in Box [Number], Bay [Number]. |

| Event ID: | 24646 |
| --- | --- |
| Type: | Warning |
| Message: | Recovery of logical drive [Number] configured on [Array controller location], was aborted while rebuilding physical drive in Box [Number], Bay [Number] due to an unrecoverable write error. The physical drive reporting the error is in Box [Number], Bay [Number]. |

| Event ID: | 24647 |
| --- | --- |
| Type: | Information |
| Message: | External Array Controller attached to [Array controller location] is now active. |

| Event ID: | 24648 |
| --- | --- |
| Type: | Information |
| Message: | External Array Controller attached to [Array controller location] is now redundant. |

| Event ID: | 24649 |
| --- | --- |
| Type: | Error |
| Message: | External Array Controller attached to [Array controller location] is no longer redundant because an array controller is missing. |
| Event ID: | 24650 |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been created. |

| Event ID: | 24651 |
| --- | --- |
| Type: | Error |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], failed and has been deleted. |

| Event ID: | 24652 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been restored. |

| Event ID: | 24653 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been detached. |

| Event ID: | 24654 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been deleted. |

| Event ID: | 24655 |
| --- | --- |
| Type: | Error |
| Message: | Restore operation of SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], is failed, possible data corruption. |

| Event ID: | 24656 |
| --- | --- |
| Type: | Warning |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been disabled. |

| Event ID: | 24657 |
| --- | --- |
| Type: | Information |
| Message: | Performing a recovery for a restore operation on SnapShot [Number] for Logical Drive [Number] on [Array controller location]. |

| Event ID: | 24658 |
| --- | --- |
| Type: | Information |
| Message: | Restore operation started on SnapShot ID [Number] for Logical Drive [Number] on [Array controller location]. |

| Event ID: | 24659 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location], has been created. |

| Event ID: | 24660 |
| --- | --- |
| Type: | Error |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location] failed and has been deleted. |

| Event ID: | 24661 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location], has been restored. |

| Event ID: | 24662 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location], has been detached. |

| Event ID: | 24663 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location], has been deleted. |

| Event ID: | 24664 |
| --- | --- |
| Type: | Error |
| Message: | Restore operation of SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location] is failed, possible data corruption. |

| Event ID: | 24665 |
| --- | --- |
| Type: | Warning |
| Message: | SnapShot ID [Number] of Logical Drive [Number] on [Array controller location], has been disabled. |

| Event ID: | 24666 |
| --- | --- |
| Type: | Information |
| Message: | Performing a recovery for a restore operation on SnapShot [Number] for Logical Drive [Number] on the external controller attached to [Array controller location]. |

| Event ID: | 24667 |
| --- | --- |
| Type: | Information |
| Message: | Restore operation started on SnapShot ID [Number] of Logical Drive [Number] on the external controller attached to [Array controller location]. |

| Event ID: | 24668 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on [Array controller location], is okay. |

| Event ID: | 24669 |
| --- | --- |
| Type: | Warning |
| Message: | Resource Volume of Logical Drive [Number] on [Array controller location], is approaching its limit. |

| Event ID: | 24670 |
| --- | --- |
| Type: | Error |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on [Array controller location], has failed. |

| Event ID: | 24671 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on [Array controller location], has been deleted. |
| Event ID: | 24672 |
| Type: | Error |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on [Array controller location], has reached its limit. |

| Event ID: | 24673 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on the external controller attached to [Array controller location], is okay. |

| Event ID: | 24674 |
| --- | --- |
| Type: | Warning |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on the external controller attached to [Array controller location], is approaching its limit. |

| Event ID: | 24675 |
| --- | --- |
| Type: | Error |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on the external controller attached to [Array controller location], has failed. |

| Event ID: | 24676 |
| --- | --- |
| Type: | Information |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on the external controller attached to [Array controller location], has been deleted. |

| Event ID: | 24677 |
| --- | --- |
| Type: | Error |
| Message: | SnapShot Resource Volume of Logical Drive [Number] on the external controller attached to [Array controller location], has reached its limit. |

| Event ID: | 24678 |
| --- | --- |
| Type: | Warning |
| Message: | The License Key on [Array controller location] is nearing its expiration date. License Key will expire in [Number] day(s). See next event (UP ARROW) for License Key |

| Event ID: | 24679 |
| --- | --- |
| Type: | Warning |
| Message: | The License Key: [Continued text string]. |

| Event ID: | 24680 |
| --- | --- |
| Type: | Error |
| Message: | The License Key on [Array controller location] has expired See next event (UP ARROW) for License Key |

| Event ID: | 24683 |
| --- | --- |
| Type: | Warning |
| Message: | SCSI bus fault occurred on Storage Box [Number], Port [Number] of [Array controller location], which may result in a "downshift" in transfer rate for one or more hard drives on the bus. |

| Event ID: | 24684 |
| --- | --- |
| Type: | Information |
| Message: | Error log update occurred for Physical Drive [Number] of [Array controller location]. |
| Event ID: | 24685 |
| Type: | Information |
| Message: | The Event Notification driver Cpqcisse.sys of [Array controller location] has started. |

| Event ID: | 24686 |
| --- | --- |
| Type: | Error |
| Message: | The Event Notification driver Cpqcisse.sys of [Array controller location] has failed to start. |

| Event ID: | 24687 |
| --- | --- |
| Type: | Information |
| Message: | The [Array controller location] has reported that its Cache has been temporarily disabled. |

| Event ID: | 24688 |
| --- | --- |
| Type: | Information |
| Message: | The [Array controller location] has reported that its Cache has been re-enabled. |

| Event ID: | 24689 |
| --- | --- |
| Type: | Information |
| Message: | The [Array controller location] has reported that its Cache batteries have been removed. |

| Event ID: | 24690 |
| --- | --- |
| Type: | Information |
| Message: | Consistency initialization pass completed for logical drive [Number] of [Array controller location]. |

| Event ID: | 24691 |
| --- | --- |
| Type: | Information |
| Message: | External redundant controller [Number], attached to the [Array controller location] has been [Action]. |

| Event ID: | 25001 |
| --- | --- |
| Type: | Information |
| Message: | Back (DOWN ARROW) message continued = [Text String]" Forward (UP ARROW) |

| Event ID: | 25002 |
| --- | --- |
| Type: | Information |
| Message: | Back (DOWN ARROW) message continued = [Text String]" END |

| Event ID: | 25003 |
| --- | --- |
| Type: | Warning |
| Message: | The event notification information received from [Array controller location] was of unknown or unrecognized class. See next event (UP ARROW) for Controller message. |