-------------------------------------------------

Season95 -- MS-DOS Videocrypt smart card emulator

-------------------------------------------------

1995-05-29 -- G - ANTWERP	     release note

INTRODUCTION

------------

Most of the information about the videocrypt protocol is found in

season7.doc written by Markus Kuhn.

This new file (SEASON95.DOC) is ment

to be a container for additional information about the videocrypt

system (cards series 09, 0a and further). Currently I have not

very much additional information yet.

The file season95.ZIP contains the season95.exe program, which is

compatible with season71.Exe. This ZIP file contains also the sources

for season95.exe.

SEASON95.EXE currently emulates a BskyB 09-series card, ECM of

17-05-1995 included.

SKY ECM 17-05-1995

------------------

new nano subcommand for the nano 11h command: subcommand 08

probably means:

switching between rompage 1 and rompage 2

HELP FOR EUROTICA AND ADULT CHANNEL

-----------------------------------

It seems to me that these 2 channels don't work with any season anymore

since 24/5/1995 (or alike). I'll upgrade SEASON95.exe if someone

can send me VCLOG information (from real cards) and/or additional hints

on how to do this. Post to alt.satellite.tv.europe

SEASON95.EXE: usage

-------------------

Syntax: SEASON95 [com] [wax] [wbx] [wcx] [wdx] [wex] [d] [e] [m]

com: which com port

wa thru we are delays, all entered in ms (milliseconds)

wa wait between reset and answer to reset

wb wait after each outgoing byte

wc wait between header and first procedure byte

wd wait between procedure byte and data

we wait between final data byte and procedure bytes

d: debug mode on when program is started

e: save time by doing less screen updates

m: OSD suppress

defaults are:

com: 1

wa: 50

wb: 0

wc: 0

wd: 0

we: 0

d: no debug

e: no time saving

m: no OSD suppression

Example: SEASON95 1 wa0 wb0 m

starts season95 using comport 1 with reset delay and byte delay set at

0 milliseconds, and with onscreen display suppressed.

For 486 and faster computers, use wa1. Experimenting with wax thru

wbx may be necessary to get the thing working.

Keyboard commands once the program is running:

F1: help -> page up, down scroll help

q: quit program

d: toggle debug mode on/off

l: write last crypto messages to logfile VCLOG

a: increase reset delay

b: decrease reset delay

+: increase byte delay

-: decrease byte delay

VERSION HISTORY

---------------

1.0: . 29-05-1995

. first implementation, derived from SEASON7.c (found in SEASON12.ARJ

on an ftp-site of the internet, don't remember which one, being

the source for season in the 07-series period) and SKY09PUB.ZIP

(found on utelscin.el.utwente.nl) being the implementation of

the 09-series for PIC controller). ECM of 17-05-1995 implemented

by debugging SEASON71.EXE

. channel name should now be correct compared to SEASON71.exe

. new user interface

1.1  .03-06-1995

. add adult and eurotica by i5uxj

1.2  .05-06-1995

. add adult and eurotica without error while SKY-Decoding

. modification on asys.asm for better operation on some computers

1.3  . 18-6-1995

. fixed old and new key on adult

. fixed 16.6.95 by i5uxj