16

CHAPTER 3 - ASMHELP

We are now going to introduce both the 8086 registers and a

program for looking at them. You are going to get information

flying at you at a rapid pace, so read both this and the next

chapter carefully and slowly.

REGISTERS

The 8086 has a number of registers. Remember, registers are

places for storing data that are internal to the 8086 chip. They

are much faster, but there are very few of them.

There are 6 registers that you can use for addition and subtrac-

tion of word (2 byte) sized numbers, as well as logical opera-

tions on word (2 byte) numbers or data. These registers are AX,

BX, CX, DX, SI, DI. In addition, there is a register which works

the same way, but has a special function in all high-level

languages (Basic, Pascal, C, etc.). This is BP, the base pointer.

There is one more register that performs the same operations as

the above seven, but it is RESERVED for special use and should

never be used for anything. It is called SP (the stack pointer).

There are 4 registers that tell the 8086 which memory segments

you are in. They just sit there and help the 8086 find things in

memory. You will learn how they work later. They are CS, DS, SS,

ES. (That is Code Segment, Data Segment, Stack Segment and Extra

Segment respectively).

There is the flags register which contains all the information

the 8086 needs to evaluate its state. We will learn about this

later. The flags register has no name, and there are machine

instructions for manipulating individual flags in the register.

Finally, there is IP, the instruction pointer, which points to

the machine instructions. You have no direct access to this,

which is good because you would screw it up for certain. The 8086

handles the IP automatically and correctly.

One word (two bytes or 16 bits) is the largest piece of data that

the 8086 can handle naturally. It is possible to handle larger

things, but we do it through software (which is slower), not

hardware (which is faster). Sometimes we want to handle things

one byte at a time as when we work with characters. The 8086

gives us this possibility by letting us divide the AX, BX, CX,

and DX registers into an upper half and a lower half. For any or

all of these registers, we can replace one 2 byte register by two

1 byte registers. The data in the full register stays the same,

but we can look at each half. The two parts of AX are called AH

(for A high) and AL (for A low).

|------AX------|

0000000000000000

|--AH--||--AL--|

Chapter 3 - ASMHELP.OBJ                                        17

_______________________

This is the 16 bit binary number 0 in the AX register. Using AX

allows us to manipulate all 16 bits. Using AH allows us to

manipulate the upper 8 bits (without affecting the lower 8 bits),

and using AL allows us to manipulate the lower 8 bits without

affecting the upper 8 bits. Similarly, for BX we have BH, BL, for

CX we have CH, CL, and for DX we have DH, DL.

SHOW_REGS

We have named all the registers, now let's take a look at them.

Included in the module asmhelp.obj is a program called show_regs.

It shows all the above registers on the top 10 lines of your

screen and allows you to enter data normally underneath.

When you call show_regs, it puts the current value of all the

registers on the screen. Those values stay on the screen until

the next call - i.e. the program does not change what is on the

screen even though the registers may be changing value. You need

to call show_regs every time that you want to see the current

values of the registers.

The first time you call show_regs, it clears the screen so you

should call it right at the beginning of the program in order to

initialize the screen.

This time we want temp2.asm for a template; we will call this

program prog3.asm, so make a copy of temp2.asm and give it the

new name.  Let's take a look at it. The only differences are (1)

there are a lot more programs in the EXTRN statements and (2) in

the data segment DATASTUFF there are these definitons:

ax_byte  db   2

bx_byte  db   2

cx_byte  db   2

etc.

These will be used for show_regs later, but you need to learn a

few assembler instructions first.

Here's our next program:

TEMP2.ASM

+ + + + + + + + START CODE BELOW THIS LINE

call show_regs

label_one:

call get_hex

call show_regs

call get_num

call show_regs

jmp  label_one

+ + + + + + + + END CODE ABOVE THIS LINE

The PC Assembler Tutor                                         18

______________________

That's all the program does. It asks for a number, then calls

show_regs to show us what is in the registers. Note that one of

the numbers is hex while the other number is decimal.

Compile this, and link it with

>link prog3+\asmhelp ;

and we're ready to go.

The program reads information in the computer to find out what

kind of monitor you have and where the screen output goes. It

then puts the register information on the top lines. If it

doesn't appear there, we have a screwup somewhere. The text

should appear in black and white, but if you have a color monitor

you can make it a blue background with white letters.{1}

*********************** SCREEN SHOT ***************************

AX  19825                             SI  00000

BX  00000                             DI  00256

CX  00255                             BP  17113

DX  02596                             SP  00508

CS  0AA4H   DS  0A54H   ES  0A24H   SS  0A34H   IP  0018H

OF   DF  IEF   TF   SF   ZF   AF   PF   CF

+    x         +    x         E            COUNT  00003

-----------------------------------------------------------------

The PC Assembler Helper   Version 1.0

Copyright (C) 1989  Chuck Nelson   All rights reserved.

Enter a two byte hex number  4df9

Enter any decimal number  +19825

Enter a two byte hex number

*****************************************************************

This is how the screen looks after entering first a hex number,

then a decimal number. The numbers in the registers will probably

be different for you. Note that AX contains the last number that

was entered. On the left side you will see the AX, BX, CX and DX

____________________

1 To make a blue background and white letters, insert the code

"call  set_blue" before the FIRST "call show_regs". i.e.:

call  set_blue

call  show_regs

label_one:

etc.

This only works if it is allowed by the video board.

Chapter 3 - ASMHELP.OBJ                                        19

_______________________

registers. For the time being, these registers will display

unsigned numbers. On the right are the SI, DI, BP and SP

registers. They are also unsigned numbers for the moment. Below

that are the segment registers CS, DS, ES, and SS and the

instruction pointer (IP). These are hex numbers and will always

be hex numbers. The bottom line has OF, DF, IEF, etc.  These are

the flags, and the marking underneath them (either a blank or

some character) tells how they are set. Finally we have COUNT.

This has nothing to do with the 8086. It is a counter that is

incremented each time you call show_regs.

Keep entering numbers and watch the registers. You will notice

that three things are changing - AX, IP and COUNT. AX has the

last number you entered and IP keeps changing. Write down the

value of IP each time it changes. It goes back and forth between

two numbers. That is because you call show_regs in two different

places in the loop, {2} and those are two different places in

memory where the 8086 is reading the machine code.

Why is AX changing? You may have wondered in prog1.asm and

prog2.asm how that information was going back and forth between

your program and asmhelp.obj. The answer is that in all the

programs in asmhelp.obj, if you need to pass information, it is

passed via register AX. This is not the normal way to pass

information. The normal way is more elegant but more complicated.

We will cover that much later. The counter, of course, increases

by 1 each time you call show_regs.

Try entering a few more numbers and then it's time to go on to

the next program.

MOVING DATA

Obviously, we want to move data from memory to the 8086, from the

8086 to memory, and between registers. We have the following

possibilities:

(1)  move from a register to memory

(2)  move from memory to a register

(3)  move a constant to memory

(4)  move a constant to a register

(5)  move from one register to another register

That's it. There is no 8086 instruction that moves a single word

or a byte from one place in memory to another.

The move mnemonic for the 8086 is "mov".{3}  We need some sample

data:

____________________

2 IP actually has three different values, since you call

show_regs once before you enter the loop.

3 A mnemonic is a name for a machine instruction, which sounds

like what the instruction is supposed to do - MOV for move, SUB

for subtract, IMUL for integer multiplication, etc.

The PC Assembler Tutor                                         20

______________________

EXAMPLE DATA

this_year     dw   1989

total         dw   ?

average       dw   ?

time          dw   7

age           db   ?         ; I hope you aren't older than 255

poem          db   "In 1492 Columbus played a mean kazoo."

secret_code   db   3Bh

character     db   ?

Here is some sample code. To move from register to memory, we

have:

mov  total, ax

mov  time, si

mov  age, cl

mov  character, bh

The first thing that strikes the eye is that the destination is

on the left and the source is on the right.{4}  This is standard

for the 8086 instruction set, and it's going to take some getting

used to. DESTINATION ON LEFT, SOURCE ON RIGHT. You are going to

blow this one from time to time, so always double check to make

sure that they are in the right order.

Also note that the 1 byte registers are matched up with 1 byte

variables, and 2 byte registers are matched up with 2 byte data.

If the sizes don't match, the assembler will complain.{5}  Thus:

mov  age, ax

is an illegal instruction.

For examples of memory to register moves, we have:

mov  ch, secret_code

mov  di, this_year

mov  dl, poem       ; moves 'I' to dl

mov  bx, time

____________________

4 The computer community likes the words "destination" and

"source". "Source" means FROM, and "destination" means TO. The

8086 instruction set is designed:

MOV  TO, FROM

which is exactly opposite to the way you would say it in an

English sentence. For the 8086, it is always destination on the

left, source on the right.

5 Half register and full register operations have different

machine codes, and the assembler needs to know which code to use,

so the two things must be the same number of bytes.

Chapter 3 - ASMHELP.OBJ                                        21

_______________________

Once again: (1) DESTINATION ON LEFT, SOURCE ON RIGHT, and (2) the

sizes of the two objects must match.

For constant to memory we have:

mov  total, 2986

mov  age, 36h

mov  secret_code, 0110100b

mov  average, (27/5) + 3

The arithmetic is done by the assembler and anything that is made

up totally of constants is legal. Thus:

mov  average, (((64+27)*51 )/(196-82))

is legal but:

mov  average, this_year/time

is illegal. The assembler makes either a one byte or a two byte

constant to match the size of the destination. The constants for

"total" and "average" are two byte constants while those for

"age" and "secret_code" are one byte constants. The constants

must be within range, that is -129 is too negative for a byte,

256 is too positive for a byte, -32769 is too negative for a

word, 65536 is too positive for a word. The assembler will give

an error if the constant is out of range.

You can also move a constant to a register:

mov  al, 'c'

mov  ax, 'c'

mov  di, 46280

mov  bl, 99

The same rules apply. The constant must be within range and the

assembler will make a constant the same size as the destination

register (one or two bytes). These constants are actually

imbedded in the machine code by the assembler, and are

unchangable.

Lastly, you can move data from one register to another:

mov  ax, cx         ; from cx to ax

mov  ah, bl         ; from bl to ah

mov  dl, dh         ; from dh to dl

mov  di, dx         ; from dx to di

All this is summarized at the end of the chapter.

It's time for program #4. All this program is going to do is get

input and the move it to different registers. We are still using

temp2.asm. Here's the program:

TEMP2.ASM

The PC Assembler Tutor                                         22

______________________

;+ + + + + + + + + + START DATA BELOW THIS LINE

byte_data  db ?

word_data  dw ?

;+ + + + + + + + + + END DATA ABOVE THIS LINE

;+ + + + + + + + + + START CODE BELOW THIS LINE

call show_regs

This_is_a_very_long_label_name:

call get_hex                       ; (1)

call show_regs_and_wait

mov  dx, ax                        ; (2)

call show_regs_and_wait

mov  byte_data, al                 ; (3)

mov  ch, byte_data

call show_regs_and_wait

mov  word_data, ax                 ; (4)

mov  di, word_data

call show_regs

jmp  This_is_a_very_long_label_name

;+ + + + + + + + + + END CODE ABOVE THIS LINE

There is a data section in this one, so copy those variables into

your data section. Here is what the program does. (1) it gets a

hex number from the keyboard, (2) it moves the number in ax to

dx, (3) it moves one byte from al to ch via the variable

byte_data, and (4) it moves two bytes from ax to di via

word_data.

There are two different subprograms - show_regs and

show_regs_and_wait. They do the same thing except that

show_regs_and_wait waits for you to hit the ENTER key before

continuing. The computer works so fast that we wouldn't be able

to see the changes in the screen if we didn't have a way of

pausing. You can use COUNT on the screen to keep track of exactly

where you are in the loop.

Assemble program 4, link it to asmhelp.obj, and watch it work.

There are two things to notice here. First, we are entering a hex

number, but AX is displaying an unsigned number. It is not

self-evident that the unsigned number in AX is the same as the

hex number that you are entering. Secondly, though CH is

changing, there doesn't seem to be any relationship between the

number in AX and the number in CX. We will solve both problems in

the next chapter.

Chapter 3 - ASMHELP.OBJ                                        23

_______________________

SUMMARY

MOVING DATA

You can:

(1)  move from a register to memory

(2)  move from memory to a register

(3)  move a constant to memory

(4)  move a constant to a register

(5)  move from one register to another register

The constants are actual constants which are imbedded in the

machine code.

REGISTERS

The normal registers are AX, BX, CX, DX, SI, DI AND BP. AX, BX,

CX, and DX can be divided into AH-AL, BH-BL, CH-CL AND DH-DL. The

'H' is for high and the 'L' is for low.

SP is committed and may not be used.

The segment registers are CS, DS, ES, SS.

The instruction pointer (IP) is not available to you.

The register that holds the flags is manipulated by special

machine instructions.

ASMHELP.OBJ

Call show_regs to see what is in the 8086 registers. Count

increments by one every time you call show_regs.

show_regs_and_wait is the same as show_regs except that it waits

for you to hit the ENTER key to allow you time to look at the

screen.

Call set_blue at the outset if you have a color card and a color

monitor and you want to have a blue background.

get_num gets a signed or unsigned number from the keyboard. (1-5

digits). It does no range checking to see whether the number is

too big. All other input routines check to see if a number is too

large for its data size.

get_hex gets a hex number from the keyboard.  (1-4 digits)

get_ascii gets characters from the keyboard.  (1 or 2 characters)

get_binary gets a binary number from the keyboard (1 - 16 digits)

print_num prints a number as hex, signed, unsigned, char, and

binary.

All data transfer to or from ASMHELP.OBJ is via the AX register.