xxvii

APPENDIX III - INSTRUCTION SPEED AND FLAGS

This appendix contains a list of all the 8086 instructions along

with their relatives speed and the flags they affect. These speed

numbers are from INTEL and are in clock ticks.{1} If you have a

25 mhz clock, then it is doing 25 million ticks per second. If

you have a 4.77 mhz clock, then it is doing 4.77 million ticks

per second. Instead of calling them ticks, I'll be calling them

clocks.

In order to understand these numbers, you need a little extra

information. In order to do an instruction, ANY microprocessor

must:

(1) calculate any memory addresses.

(2) fetch the two operands (or the single operand if it is

an instruction like NOT).

(3) process the instruction and

(4) put the result in the destination.

While (3) will require the same amount of time whether the

operands are in memory or in registers, (1), (2) and (4) are all

different depending on where the operands are. Some machines

allow both operands to be in memory, but the 8086 does not.

Therefore, (1) is either NO memory addresses (if everything is in

registers) or ONE memory address.

Calculating a memory address involves (a) calculating the offset

from the beginning of the segment and (b) adding it to the

segment starting address. Part (a) is different depending on the

addressing mode. In terms of the speed of calculating (a) and

(b), the order is:

(i)     one pointer

(ii)    named variable (+ constant)

(iii)   two pointers

(iv)    one pointer + constant

(v)     two pointers + constant

The reason that (ii) contains both "variable" and "variable +

constant" is that the addition (variable + constant) is done by

the assembler, not the 8086. By the time the 8086 sees it, it is

a single number. The constants in both (iv) and (v) need to be

added by the 8086, and this takes extra time. According to INTEL,

here is the time required for calculating all possible memory

addresses. Note that some pointers are marginally faster than

other pointers (this is trivial - don't worry about it).

____________________

1 All the speed numbers are from "Programmer's Pocket

Reference Guide", (c)1980, 1982 Intel Corporation.

______________________

The PC Assembler Tutor - Copyright (C) 1990 Chuck Nelson

The PC Assembler Tutor                                     xxviii

______________________

ADDRESSING MODE                          CLOCKS (EA)

(i)   [bx], [si], [di], [bp]                           5

(ii)  variable (+ constant)                            6

(iii) [bp+di] or [bx+si]                               7

[bp+si] or [bx+di]                               8

(iv)  ([bx] or [si] or [di] or [bp]) + constant        9

(v)   ([bp+di] or [bx+si]) + constant                  11

([bp+si] or [bx+di]) + constant                  12

The most complicated memory address takes 2.4 times longer to

calculate than the simplest address. These calculation times will

be noted by EA (calculate Effective Address). Remember, if both

operands are in registers, this calculation does not have to be

done.

In order for you to see how all of this works, we'll use ADD as

an example. Don't start using the table till you understand this

example.

On the 8086, we normaly have the following possibilities for

source and destination:

register, register

register, memory

memory, register

register, constant

memory, constant

In Appendix II, we simply combined them as:

reg/mem, reg/mem

reg/mem, constant

We can't do this here because they all have different times. For

ADD they are:

ADD                      CLOCKS

register, register       3

register, memory         9  + EA

memory, register         16 + EA

register, constant       4

memory, constant         17 + EA

Notice how much faster using a register is. The EA stands for

"calculate Effective Address" and is the number from the list

above. If you have:

add  ax, bx

that is "register, register", and it will take 3 clocks to

execute. If you have:

add [bx+di+9], 17

that is "memory, constant" and will take 17 + EA. What is EA

here? According to the above list, [BX+DI+CONSTANT] takes 12

Appendix III - Speeds and Flag Settings                      xxix

_______________________________________

cycles, so 17 + EA is 17 + 12 is 29, so this will take 29 clocks.

That's right. The one instruction is almost 10 times slower than

the other. If you can move things into some registers, do a

number of calculations, and then move them back to memory, you

can save a lot of time. Let's do a few examples to make sure that

you see all of them:

INSTRUCTION              TYPE                TIME

add  variable1, bl       memory, register    16 + EA = 22

add  bl, variable1       register, memory    9  + EA = 15

add  [si], di            memory, register    16 + EA = 21

add  di, [si]            register, memory    9  + EA = 14

Examples 1 and 2 are the same except that source and destination

have been switched. The same applies to examples 3 and 4. Notice

that when the 8086 has to fetch the variable and then put the

result back in memory, it is significantly slower than when it

just gets the variable from memory and puts the result in a

register.

INSTRUCTION              TYPE                TIME

add  ax, cx              register, register  3

add  di, 1876            register, constant  4

add  variable1, 199      memory, constant    17 + EA = 23

To show you how the different types of EA effect the time, let's

do all 3 types of "source, destination" that involve memory.

First, "memory, register":

INSTRUCTION              TIME

add  [bx], ax            16 + EA = 21

add  variable1, ax       16 + EA = 22

add  [bp+di], ax         16 + EA = 23

add  [bp+si], ax         16 + EA = 24

add  [bx+9], ax          16 + EA = 25

add  [bp+di+294], ax     16 + EA = 27

add  [bx+di+294], ax     16 + EA = 28

Now let's do the same things but go "register, memory":

INSTRUCTION              TIME

add  ax, [bx]            9 + EA = 14

add  ax, variable1       9 + EA = 15

add  ax, [bp+di]         9 + EA = 16

add  ax, [bp+si]         9 + EA = 17

add  ax, [bx+9]          9 + EA = 18

add  ax, [bp+di+294]     9 + EA = 20

add  ax, [bx+di+294]     9 + EA = 21

The PC Assembler Tutor                                        xxx

______________________

And finally we have "memory, constant":

INSTRUCTION              TIME

add  [bx], 177           17 + EA = 22

add  variable1, 177      17 + EA = 23

add  [bp+di], 177        17 + EA = 24

add  [bp+si], 177        17 + EA = 25

add  [bx+9], 177         17 + EA = 26

add  [bp+di+294], 177    17 + EA = 28

add  [bx+di+294], 177    17 + EA = 29

Is this everything you need to know before looking at the list?

Not quite. Most of the 8086 family has a 16 bit data bus. That

means that there are 16 wires connecting the processor to memory,

and the processor reads 1 word (2 bytes) at a time. These memory

reads ALWAYS start at an even location 1472d, 88026d, 198752d,

etc. If you are reading one byte, it makes no difference whether

it is at an even or odd location. If you are reading a word at an

even location, then everything is normal. If you are reading a

word at an ODD location, however, the processor must:

1) start reading at the first even location that contains

the variable.

2) read the next even location (which contains the last part

of the variable).

3) join the parts together.

As an example, let's take a word at address 21957 (i.e. 21957-

21958). The processor will:

1) read the high byte from the word at 21956

2) read the low byte from the word at 21958

3) join them together. It now has 21957-21958.

The processor can do this, but it takes extra time (4 extra clock

ticks), so our speed listing will also contain the following

notice:

WORDS WHICH ARE AT ODD ADDRESSES NEED 4 EXTRA CLOCKS

Thus for:

add  ax, variable1        (9 + EA = 15)

if the address is an even location, the instruction will require

15 clocks. If it is an odd location, the instruction will require

19 clocks (4 extra ticks). It is worth your while to keep words

at even locations if at all possible.

Appendix III - Speeds and Flag Settings                      xxxi

_______________________________________

INSTRUCTION SPEEDS AND FLAGS

REGISTER:

One of the arithmetic registers - either word (AX, BX, CX,

DX, SI, DI, BP, SP for word operations) or byte (AH, AL, BH,

BL, CH, CL, DH or DL for byte operations).

AX or AL:

AX and AL are considered special on the 8086. Sometimes

there is a special AX/AL form of the instruction, (such as

for ADD). This form will be shorter. It will only be noted

if it is FASTER (such as for MOV) or if it is the only form

allowed. It will either say (AX/AL) or (AX only) depending

on whether both words and bytes are allowed or only words

are allowed. Though both multiplication and division require

the use of the (AX/AL) register, the AX/AL is understood,

and is not mentioned in the instruction.

SEGREG:

One of the 4 segment registers - CS, DS, ES or SS.

MEMORY:

Either a byte or word in memory. It may be addressed with

any possible addressing mode, and the extra time needed to

calculate the address in memory (EA - calculate Effective

Address) is the following:

ADDRESSING MODE                          CLOCKS (EA)

(i)   [bx], [si], [di], [bp]                           5

(ii)  variable (+ constant)                            6

(iii) [bp+di] or [bx+si]                               7

[bp+si] or [bx+di]                               8

(iv)  ([bx] or [si] or [di] or [bp]) + constant        9

(v)   ([bp+di] or [bx+si]) + constant                  11

([bp+si] or [bx+di]) + constant                  12

EXTRA TIME:

1) WORDS WHICH ARE AT ODD ADDRESSES NEED 4 EXTRA CLOCKS.

2) A segment override adds 2 clocks to the instruction.

FLAGS

Following the instruction mnemonic is information in square

brackets that indicates how the instruction effects the flags

register. There are three possibilities

1) The instruction may alter the value of a flag in a

particular way depending on the result. For AND, the sign,

zero and parity flags [SZP] will be set according to the

result.

The PC Assembler Tutor                                      xxxii

______________________

2) The instruction may set a flag to a specific number

(either 1 or 0). For AND, the overflow flag and the carry

flag are cleared [(OC=0)].

3) The instruction may unreliably alter a flag. That means

that the instruction might change the flag, but that it

gives you no information. This kind of flag cannot be

trusted after this operation. For AND, the auxillary flag is

unreliable [?A?].

The information will always be displayed in this order. The flags

which are reliably set according to the result will be listed

first [SZP]. Next, any flags which are either set or cleared will

be put inside parentheses followed by an equal sign followed by

the value of the flag (all inside of the parentheses)

[SZP,(OC=0)]. Finally, any unrelaible flags will be put between

question marks [SZP,(OC=0),?A?]. Each part will be separated by

commas. If no flags are changed by the instruction, the brackets

will have [none] written between them.

Each flags will be indicated by a single letter. The letters are:

O    overflow flag       0 = no overflow, 1 = overflow

D    direction flag      direction of movement

for string instructions

0 = upwards, 1 = downwards

I    interrupt enable    0 = no ints, 1 = ints o.k.

T    trap flag           trap next instruction?

0 = no trap, 1 = trap

S    sign flag           0 = positive, 1 = negative

Z    zero flag           0 = non-zero, 1 = zero

A    auxillary flag      carry out of bottom half

register?  0 = no, 1 = yes

P    parity flag         0 = odd, 1 = even

C    carry flag          0 = no carry, 1 = carry

***************** THE INSTRUCTIONS *********************

INSTRUCTION                TIMING

AAA [AC,?OSZP?]         4 clocks

AAD [SZP,?OAC?]         60 clocks

AAM [SZP,?OAC?]         83 clocks

AAS [AC,?OSZP?]         4 clocks

Appendix III - Speeds and Flag Settings                    xxxiii

_______________________________________

ADC [OSZAPC]            see ADD

ADD [OSZAPC]            register, register    3

register, memory      9  + EA

memory, register      16 + EA

register, constant    4

memory, constant      17 + EA

AND [SZP,(OC=0),?A?]    see ADD

CALL [none]             near call                19

far call                 28

near call (reg-ind)      16  {2}

near call (mem-ind)      21 + EA

far call (mem-ind)       37 + EA

CBW [none]              2 clocks

CLC [(C=0)]             2 clocks

CLD [(D=0)]             2 clocks

CLI [(I=0)]             2 clocks

CMC [C]                 2 clocks

CMP [OSZAPC]            register, register    3

register, memory      9  + EA

memory, register      9  + EA

register, constant    4

memory, constant      10 + EA

CMPS [OSZAPC]           22 clocks

CWD [none]              5 clocks

DAA [SZAPC,?O?]         4 clocks

DAS [SZAPC,?O?]         4 clocks

DEC [OSZAP]             word register       2

byte register       3

word/byte memory    15 + EA

DIV [?OSZAPC?]          byte register       80 - 90  {3}

word register       144 - 162

____________________

2 These three last ones are indirect calls. They get the

address of the subroutine from a register (reg-ind) or from

memory (mem-ind).

3 The smaller the numbers, the faster this operation can be

accomplished. This applies for signed and unsigned multiplication

and division. They all show a range of values rather than a

specific value.

The PC Assembler Tutor                                      xxxiv

______________________

byte memory         (86 - 96) + EA

word memory         (150 - 168) + EA

ESC [none]              memory              8 + EA

coproc. register    2

This only makes sense if you know

about coprocessors.

HLT [none]              2 clocks

IDIV [none]             byte register       101 - 112

word register       165 - 184

byte memory         (107 - 118) + EA

word memory         (171 - 190) + EA

IMUL [OC,?SZAP?]        byte register       80 - 98

word register       128 - 154

byte memory         (86 - 104) + EA

word memory         (134 - 160) + EA

IN [none]               (AX/AL), port#      10

(AX/AL), dx         8

INC [OSCAP]             word register       2

byte register       3

word/byte memory    15 + EA

INT [(IT=0) {4} ]       51 clocks  {5}

INTO [ {6} ]            overflow            54

no overflow         4

IRET [ {7} ]            24 clocks

J(condition) [none]     This includes all conditional jumps

(JAE, JZ, JNO, JLE, JP, etc.) with the

exception of JCXZ.

jump                16

no jump             4

____________________

4 Although this sets the trap flag and interrupt flag to 0, it

doesn't do it to YOUR flags, it does it to the flags that the

interrupt will see. Your flags are safely stored on the stack and

will return unaltered at the end of the interrupt.

5 There is one exception. INT 3, when coded as the single byte

trap interrupt, is 52 clocks.

6 If there is overflow, it pushes your flags and sets (IT=0)

for the interrupt. If there was no overflow, it does nothing. In

either case your own flags will remain unaffected.

7 This puts the copy of your old flags back in the flags

register. The flags will be the same as they were when you called

the interrupt.

Appendix III - Speeds and Flag Settings                      xxxv

_______________________________________

JCXZ [none]             jump                18

no jump             6

JMP [none]              same segment        15

different segment   15

near (reg-ind)      11  {8}

near (mem-ind)      18 + EA

far  (mem-ind)      24 + EA

LAHF [none]             4 clocks

LDS [none]              16 + EA

LES [none]              16 + EA

LEA [none]              2 + EA

LOCK [none]             2 clocks

LODS [none]             12 clocks

LOOP [none]             jump                17

no jump             5

LOOPE/LOOPZ [none]      jump                18

no jump             6

LOOPNE/LOOPNZ [none]    jump                19

no jump             5

MOV [none]              register, register    2

register, memory      8  + EA

memory, register      9  + EA

register, constant    4

memory, constant      10 + EA

(AX/AL) <-> memory    10 {9}

____________________

8 These last three are indirect jumps. The information about

where to jump to is coming from a register (reg-ind) or from

memory (mem-ind).

9 This is a special instruction which moves a directly

addressed variable to or from AX (or AL for bytes). Pointers are

not allowed, only the forms:

mov  ax, variable1

mov  variable1, ax

This takes 10 clocks instead of 14 or 15 for the other form.

Whether this form gets used is up to the assembler, not you.

Fortunately, MASM, TurboAssembler and A86 all use this form when

appropriate.

The PC Assembler Tutor                                      xxxvi

______________________

segreg <-> register   2  {10}

segreg, memory        8 + EA

memory, segreg        9 + EA

MOVS [none]             11 clocks

MUL [OC,?SZAP?]         byte register       70 - 77

word register       118 - 133

byte memory         (76 - 83) + EA

word memory         (124 - 139) + EA

NEG [OSZAPC] {11}       register            3

memory              16 + EA

NOP [none]              3 clocks

NOT [none]              register            3

memory              16 + EA

OR [SZP,(OC=0),?A?]     see ADD

OUT [none]              (AX/AL), port#      10

(AX/AL), dx         8

POP [none]              register            8

segreg              8

memory              17 + EA

POPF [ {12} ]           8 clocks

PUSH [none]             register            11

segreg              10

memory              16 + EA

PUSHF [none]            10 clocks

RCL [OC]                register by 1 bit     2

memory by 1 bit       15 + EA

register by # in CL   8 + (4 * #) {13}

memory by # in CL     20 + EA + (4 * #)

RCR [OC]                see RCL

____________________

10 MOV, PUSH and POP are the only instructions that can alter

the segment registers (other than CALLs and JMPs).

11 (NEG number) sets the flags the same as (SUB 0, number).

12 POPF resets the flags register by POPping a word of the

stack and using the values stored in that word.

13 Thus, if you rotate right by 3 bits add (4 * 3), if you

rotate by 7 bits add (4 * 7), if you rotate by 2 bits add

(4 * 2).  As you can see, this can cost a lot of time if you are

rotating more than 3 or 4 bits.

Appendix III - Speeds and Flag Settings                    xxxvii

_______________________________________

REP [none]              2 clocks

RET [none]              near ret            8 {14}

near ret (#)        12

far ret             18

far ret (#)         17

ROL [OC]                see RCL

ROR [OC]                see RCL

SAHF [ {15} ]           4 clocks

SAL/SHL [OSZPC,?A?]     see RCL

SAR [OSZPC,?A?]         see RCL

SBB [OSZAPC]            see ADD

SCAS [OSZAPC]           15 clocks

SEGMENT OVERRIDE [none] 2 clocks

SHR [OSZPC,?A?]         see RCL

STC [(C=1)]             2 clocks

STD [(D=1)]             2 clocks

STI [(I=1)]             2 clocks

STOS [none]             11 clocks

SUB [OSZAPC]            see ADD

TEST [SZP,(OC=0),?A?]   register, register    3

register, memory      9  + EA

memory, register      9  + EA

(AX/AL), constant     4

register, constant    5

memory, constant      11 + EA

WAIT [none]             3 clocks minimum, then check every 5

clocks

XCHG [none]             register, register    4

(AX only), register   3

____________________

14 The # here indicates that you pop things off the stack as

you would in a Pascal program:

ret (18)

ret (6)

15 Alters the values of the SZAPC flags according to the

values in the AH register.

The PC Assembler Tutor                                    xxxviii

______________________

register, memory      17 + EA

XLAT [none]             11 clocks

XOR [SZP,(OC=0),?A?]    see ADD