SHOW87

------

Copyright (c) 1988, 1993 Borland International  All Rights Reserved

TABLE OF CONTENTS

-----------------

1. What is SHOW87?

2. Starting SHOW87

3. What SHOW87 Displays

1. WHAT IS SHOW87?

------------------

SHOW87 is a memory-resident program that displays the entire state of an

installed 8087 coprocessor chip when an 8087 is present. This is useful

when you're debugging code that contains 8087 instructions.

2. STARTING SHOW87

------------------

SHOW87 has two modes of installation. The default is the shell mode in

which SHOW87 executes a DOS shell, rather than making itself truly resident.

This allows you to de-install SHOW87 by typing EXIT at any DOS prompt. The

disadvantage of the shell mode is that a second command processor must be

invoked, which uses an extra 3,000 to 4,000 bytes of memory. The other

installation mode is the resident mode, which is specified by the /r

parameter. SHOW87 uses less memory in this mode, but cannot be removed.

SHOW87 alone uses about 5,900 bytes of memory.

Once installed, SHOW87 can be invoked at any time by pressing Alt-7.

Invoking SHOW87 causes most of the upper half of the screen to display the

flags, registers, and other information regarding the 8087. Pressing any key

exits you from the display and restores the screen. Normally, you would

install SHOW87 before debugging 8087 code and then remove SHOW87 when it's

no longer needed.

If SHOW87 cannot properly install itself, it will display an error message

and terminate. The most common reasons for such an error are insufficient

memory, COMMAND.COM not found, or no 80x87 is detected. COMMAND.COM is only

needed when SHOW87 is run in shell mode. SHOW87 finds the COMMAND.COM file

by looking for the COMSPEC parameter in the environment (see your DOS manual).

3. WHAT SHOW87 DISPLAYS

-----------------------

SHOW87 displays all 8087 information, including the instruction pointer, the

operand pointer, the operation code; the control, status, and tag words; the

precision, rounding, and infinity control settings; the stack top; the

condition codes and their various interpretations; the exception settings

and interrupt mask settings; and the register values.

The condition code settings represent C3, C2, C1, and C0 respectively.

The Comp, Test, and Exam fields display the meaning of the condition codes

as returned by the FCOM, FTST, and FXAM instructions.

Register values are displayed in one of two ways. If the number has a tag

setting of VALID, the number is displayed in decimal format. If the number

has a tag setting of SPECIAL or EMPTY, a hexadecimal dump of the number is

displayed. After the mantissa and exponent, the type of value (as

interpreted by the FXAM instruction) is displayed.