************************************************************************

Oki 900 4711 Mod Docs

************************************************************************

Change ESN

~~~~~~~~~~

Press MENU 8 times, until you see ADM menu.  Press RCL and enter 123456

Use RCL to move from one ESN to another, and STO to save your options.

REBOOT Phone!!!!!

Program ESN:

~~~~~~~~~~~

Enter Debug Mode:

7  9

MENU

SEND

END

RCL

STO

CLR

1  3

--------------------------------

ESN Number     Address

--------------------------------

ESN Location #1  $BE8E-$BE91

ESN Location #2  $BE93-$BE96

ESN Location #3  $BE98-$BE9B

ESN Location #4  $BE9D-$BEA0

ESN location #5  $BEA2-$BEA5

--------------------------------

+--------------------+----------------------+----------------------+

|                    |                      |                      |

| To program ESN #1: |   To program ESN #2: |  To program ESN #3:  |

|                    |                      |                      |

|   #54 BE8E  01     |     #54 BE93  01     |    #54 BE98  01      |

|   #54 BE8F  02     |     #54 BE94  02     |    #54 BE99  02      |

|   #54 BE90  03     |     #54 BE95  03     |    #54 BE9A  03      |

|   #54 BE91  04     |     #54 BE96  04     |    #54 BE9B  04      |

|                    |                      |                      |

+--------------------+----------------------+----------------------+

|                         |                         |

|   To program ESN #4:    |    To program ESN #5:   |

|                         |                         |

|     #54 BE9D  01        |       #54 BEA2  01      |

|     #54 BE9E  02        |       #54 BEA3  02      |

|     #54 BE9F  03        |       #54 BEA4  03      |

|     #54 BEA0  04        |       #54 BEA5  04      |

|-------------------------|-------------------------|

Key:

#54 XXXX xx

|   |   |

|   |  In order, one thru four bytes of ESN

|   |

|  Address (Location)

|

The write byte debug command

To use the 0-9 keys, just use 0-9, to access A-F, hit STAR ("*")

before 1-6 for A-F.  The "*" key can be thought of as a shift key.

If you hit the "*" twice, it will at as if you did not hit the "*"

at all.

*1 = A

*2 = B

*3 = C

*4 = D

*5 = E

*6 = F

Program NAM:

~~~~~~~~~~~~

RCL MENU

STAR

6

2

7

2

9

8

5

4

#

(At this point use the up and down arrow keys on the side of the phone)