Page 2-1

DEMO.DOC

DEMONSTRATION

First be sure you have made a backup of the distribution disks.  If you are not

familiar with backup procedures, then this program may have less documentation

than you will require, still why not try.

To make a backup:

1. Boot the system.  The system disk will be in drive A

2. Type DISKCOPY A: B:

3. In answer to the prompt load the distribution copies of DROEGE one

at a time in drive A and new disks in drive B and hit any key as

instructed.  It is always good practice to write protect the disk

you are copying from by covering the notch in the side of the disk

with a protection tape.

The copy process should end with the A prompt after you have answered N to the

copy another? prompt.  Replace the system disk in drive A and switch to the B

disk by typing B: (Return).  Leave the new copy in drive B which contains the

files listed below and you are ready to start.  You can check the directory by

dir(return) when you have the B: prompt.

This session is designed to give an overview of the design capacity of the

DROEGE program.  You will need the disk with the following files:

DROEGE.EXE

BRUN20.EXE

SETUP.PCD

SETUP.PCS

DEMO.PCD

DEMO.PCS

Besides DROEGE you will always need BRUN20.EXE, SETUP.PCD, and SETUP.PCS to

run.  The disk containing these files can be removed after the program puts up

its first menu.  You would normally change to your job storage disk at this

point.  For this demo, you can run with the disk (disk 1) which contains the

above files.

First some conventions.  In general, the line editing functions work in the

standard way.  That is you can use the back arrow to delete mistyped characters

etc..  Most operations with DROEGE are done using the Function keys at the left

of the IBM PC keyboard, which are designated F1 - F10.  Cursor movement is

controlled by the number keypad arrow keys at the right of the keyboard.  We

designate these KP1-KP9.  We will indicate a particular keypress by *** Key in

this demonstration, followed by the function menu that appears at the bottom of

the screen after the keypress.  Some items will be entered from the keyboard.

We will indicate this by xxxxxx (ENTER), to indicate some keyboard entry

followed by a carriage return.

We suggest that you operate with a system disk in drive A and the DROEGE disk

in drive B, with B the default drive.  Hard disk owners have a number of

options.  Place the above disk in the default drive and type:

DROEGE (ENTER).         1ENTER  2FILE   3PLOT   4SETUP  5SYMLIST

The disk will whir for a few seconds and a copyright notice and the registered

Page 2-2

owners name will appear.  After you have seen the notice, hit any key and in a

few seconds more the screen will turn blue(if you have a color monitor - you

must at least have a color adapter).

You will see a status line at the top of the screen and a command line at the

bottom of the screen.  Since we want a demonstration file ask for the file

system by pressing the 2FILE function key:

*** F2          1PUTFIL 2GETFIL 3       4LSTSET 5

This causes presentation of a new command line and a change in the status line

to show that we are now at level 2 of the command tree.  Since we want get a

demonstration file we press the 2GETFIL function key:

*** F2          1LODJOB 2LODLIB 3LSTSYM 4LSTLAB 5LSTKEY

We want to load a job (more about jobs and libraries later) so we press the

1LODJOB function key:

*** F1

We now get a message on the second screen line asking us to enter the requested

file name.  We want the file DEMO so type:

DEMO (ENTER)   1LODJOB 2LODLIB 3LSTSYM 4LSTLAB 5LSTKEY

The disk will spin for about 3 minutes (less if you have an AT) and then there

will be a ten second pause while the program links the loaded symbols.  A

circuit will now start to draw at high magnification.  We start with this

display because it looks nice.  You can see most of one IC and a few other

components.  We would normally only work at this magniffication to check for

clearances.

We can draw our layout on the screen with arbitrary scale.  Select redraw by

pushing  F10.

*** F10         1UP 7   2DWN 7  3SELECT 4SETUP  5ENTER

A menu of previously set up layout views appears.  We can set up 15 such views.

Now let us look at the big picture.  To select the big picture we use F1 and F2

to move up and down the display menu.  We have preset and named various areas

of the circuit area.  Later we will tell you how to set up display areas to

suit your design preferences.  The closest rout to the big picture is to hit F2

once.  14 F1's would also get you there.  Note that the number on the key

changes to indicate the layout we have selected.

*** F2          1UP 6   2DWN 6  3SELECT 4SETUP  5ENTER

We now have the number of the item we want so we select it:

*** F3          1LODJOB 2LODLIB 3LSTSYM 4LSTLAB 5LSTKEY

The screen now clears and the entire circuit is drawn.  This scale is too small

for data entry purposes but is a good scale for planning the layout.  Later we

will show you how to turn off some of the levels to reduce the clutter.  For

now we see a small circuit with 14 IC's, 10 resistors, 10 capacitors and a

connector.    You will see funny lines hanging off the bottom of the layout.

This is because it interconnects with another part of the circuit and these

Page 2-3

components were entered with the two parts put together.

For practical work we use an intermediate scale between the two shown so far.

We will look at the upper left hand corner.

*** F10         1UP 6   2DWN 6  3SELECT 4SETUP  5ENTER

*** F2

*** F2

*** F2

*** F2          1UP 2   2DWN 2  3SELECT 4SETUP  5ENTER

*** F3          1LODJOB 2LODLIB 3LSTSYM 4LSTLAB 5LSTKEY

This layout is a good working size.  If we have a larger job, we tend to break

it up into working units of this size.  Larger units take too long to redraw

(we are working on that) and smaller units require too much work to connect

them to their neighbors.  With more resolution we would put more parts on the

screen.  The upcomming licensed version will support the EGA which provides

four times the number of pixcells.    This layout is part of a larger layout

which we will now examine.  This in turn is part of a still larger board which

is too large to fit in the memory of this free version.  We will now travel

some distance through the key tree.  In case you get lost, keep hitting F9

until the machine beeps, then answer anything but yes and you will be at the

menu:

1ENTER  2FILE   3PLOT   4SETUP  5SYMLST

You should then be able to pick up the trail through the demonstration.  When

it seems obvious we will now omit the command line.

*** F9

*** F9          1ENTER  2FILE   3PLOT   4SETUP  5SYMLST

You may recognize this at the entry command menu.  We will move into the menu

structure to look at a symbol.

*** F1

*** F3

*** F3

*** F5

After F5 the machine beeps and we are asked to enter a symbol name.  Upper and

lower case are distinguished in symbol names so be sure to type in upper case:

STRIP (ENTER)

The screen clears. You may recognize the first part of the circuit drawn as the

previous layout.  You will notice that it is rotated by 90 degrees.  This

circuit is a mixture on a number of different techniques.  It includes CMOS,

LSTTL, VMOS, Bipolar, discrete components, a transformer, and even a relay.

The left end is a digital bus with D/A converters.  The middle section contains

operational amplifies which control the high voltage power supply at the right

hand end of the board.  The circuit produces 8 killovolts at 2 ma.  and is

stable to 100 mv. RMS.  Twelve different voltages are read back through an

analog bus to a 16 bid A/D converter where signals are measured to 100

microvolt precision.  The output voltage is controlled to within 100 millivolts

of the selected value.

You may explore this symbol by using F10 and selecting one of the setups

starting with HV.   The layout contains 25 integrated circuits, 110 other

Page 2-4

components and 600 holes.  Since the circuit takes two minutes to redraw on an

XT you may wish to use the F9 key to abort a redraw while exploring.  This

layout is composed of 28 different symbols which are nested to level 4.  That

is some symbols call other symbols to level four.  This layout contains only

the symbols:

HVBOTTOM

HVSTRIP

DIG-DAC

These symbols are in turn composed of other symbols.  Let us explore back to

see how this is done.

*** F9

*** F9 ... Enough times until the machine beeps and asks if you want to exit

the symbol.  Answer y.  The machine beeps again and tells you it has

closed the symbol.

*** F9          1NEW    2*MAKE  3ORIGIN 4KILL   5OPEN

*** F5

In answer to the question enter "PAD62S"

PAD62S (ENTER)

A square pad appears at the center of the screen.  It is quite large.  When we

open a symbol we automatically scale it to fill the screen.  While it looks

like a simple square if we used the machine facilities to examine it we would

find that it actually consists of four objects.  It is a 62 mil square pad on

layer one, which we arbitrarily define as the top side of the board.  It is

also a 62 mil pad on side 2, which we call the board bottom.  It is a 15 mil

drill spot of layer 3 which contains the drill information.  Lastly it is an

eighty mil pad on level 5 which we define as the solder mask.  If it could, it

would have  an outline on level 4.  These are strictly our internal

conventions.  As a user you are free to use the 12 layers any way you want.

Now we will see how this is used in another symbol.

*** F9 ... y to question

*** F5

In answer to question enter "IC14U"

IC14U (ENTER)

Now we get a 14 pin IC outline.  This was constructed from 14 of the previous

symbols and a line on level 5 to show the package outline.

*** F9 ... y to question

*** F5

In answer to question enter "IC4073"

IC4073 (ENTER)

Now we see a new symbol made from the previous symbol.  IC14U is our general

Page 2-5

purpose 14 pin circuit.  It is sort of like the old printed symbols use for

tape layout.  Now we have customized this one to parallel it's three internal

gates to drive a high power VMOS transistor.  We did this because we were not

quite sure where we were going to place it and we wanted to drag around its

interconnections with it.

*** F9 ... y to question

*** F5

In answer to question enter "HVBOTTOM"

HVBOTTOM (ENTER)

The high voltage section of the board now appears.  You can find IC4073 at the

lower left of this circuit.

This is the end of this demonstration.  You may hit F9 enough times, answering

y to the questions to get back to the system.

Later you may want to examine ALRM as above to look at a 1000+ component job.

*** DROEGE (RETURN)  To load DROEGE from system

*** F2               To select file system

*** F2               To get a file

*** F2               To load a library (here insert disk with ALRM.LIB)

*** ALRM (RETURN)    To load library ALRM

It will now take about  2.5 minutes to load and link this large board.

*** F9

*** F9               To get back to the main menu.

*** F1               Choose enter

*** F3               Choose symbol

*** F3               Choose create

*** F5               Choose open

ALRM (RETURN)        To open the symbol ALRM - remember to enter in upper case.

Depending on your computer and the particular version of DROEGE it will take up

to 10 minutes to redraw this 12.5" by 16.6" circuit board.  It should be

obvious that this is an unworkable scale and that the redraw time is too long

for this to be a useful program.  The way we proceed on a large board like this

is to break it up into a number of symbols.  We work on the interconnections

between symbols with only one or two symbols on the board, and at some usable

scale.  Now the redraw time is only a few seconds.  When we have completed

interconnecting a symbol, we remove it, making a note of its location.  At the

very end we put all the symbols back on the board and we are done.  So that we

almost never redraw the entire job.

*** F9               To back out of the symbol

Y (RETURN)           To confirm exit from symbol

*** F9

*** F9

*** F9               And we are back to the main menu

*** F2               To do something with files

*** F2               Choose getfil

*** F3               LSTSYM will provide a symbol listing on the line printer

of all the symbols used in making the board ALRM

*** F9

*** F9               Gets us back to the main menu.  We can now use the same

Page 2-6

sequence as we used to open the symbol ALRM to open and

examine any of the subsymbols used in ALRM

*** F1

*** F3

*** F3

*** F5

AMP (ENTER)          For example will get us the amplifier which is repeated 64

times on the ALRM board.

The next section, TUTOR.DOC, will lead you through entering lines and creating

symbols.

Page 2-7