'***********************************************************

'*	            RC FAILSAFE SOFTWARE                     *

'*                       1/30/03                           *

'***********************************************************

$crystal = 8000000

Config Timer0 = Timer , Prescale = 1024 'missing pulse timer

On Timer0 Mispulse                      'missing pulse routine

Config Pinb.3 = Output

Config Pind.6 = Input

Dim Position As Byte

Dim D As Bit

Dim X As Byte

Dim Count As Word

D = 0

X = 0

Position = Pinb                         'read position value

Position = Position And &B00000111      'use only  the 3 lsb

'*********************************************************

'this routine will set the pulse-width(servo position) using jumper info

'*********************************************************

Select Case Position

Case 0 : Pwm1a = 96              'center position 1.52 ms

Case 1 : Pwm1a = 125             'full reverse 2.0ms

Case 2 : Pwm1a = 65              'full forward 1.0 ms

Case 3 : Pwm1a = 75              '2/3 forward

Case 4 : Pwm1a = 85              '1/3 forward

Case 5 : Pwm1a = 103             '1/3 reverse

Case 6 : Pwm1a = 113             '2/3 reverse

Case 7 : Pwm1a = 96             'center position 1.52 ms

End Select

Enable Timer0

Enable Interrupts

Timer0 = 0                              'clear timer 0

Tifr.1 = 1                              'clear timer0  interrupt flag

'**********************************************************

'this routine will toggle the servo output to match the servo input

'if Tx sixnal is missed for 32ms, timer0 will overflow calling mispulse

'the config timer1 is needed to disconnect the PWM

'**********************************************************

Do

Config Timer1 = Pwm , Pwm = 10 , Compare A Pwm = Disconnect , Prescale = 64

Again:

Count = 0                               'reset pulse width count

Getpulse_3:

sbis pind,6                             'wait for signal to go high

rjmp getpulse_3

Set Portb.3                             'set output high

Getpulse_4:

Incr Count                              'one count=2.58ms

sbic pind,6                             'wait for signal to go low

rjmp getpulse_4

If Count > 855 Or Count < 350 Then      'check to be in 0.9ms-2.2ms

Goto Again

End If

Reset Portb.3                           'set output low

Timer0 = 0                              'reset timer 0

D = 0                                   'reset interrupt counter

Count = 0

Loop

End                                     'end program

'***********************************************************

'this routine will connect the PWM output to pinb.3 when Tx signal is lost

'overflow in 32ms.  If signal lost for 64ms PWM signal is connected

'***********************************************************

Mispulse:

If D = 1 Then                           'check for second interrupt

Config Timer1 = Pwm , Pwm = 10 , Compare A Pwm = Clear Down , Prescale = 64

Tryagain:

X = 0

Count = 0

For X = 0 To 4                       'check for 5 pulses before exiting

Getpulse_1:

sbis pind,6

rjmp getpulse_1

Getpulse_2:

Incr Count                           'each count= 2.58us

sbic pind,6

rjmp getpulse_2

If Count > 855 Or Count < 350 Then

Goto Tryagain

End If

Count = 0

Next X

Count = 0

D = 0                                'reset interrupt counter

X = 0                                'reset pulse counter

Timer0 = 0                           'rest timer

Tifr.1 = 1                           'reset interrupt flag

Else

D = 1                               'mark interrupt entry

End If

Return