Tech reviewer:

Please be extra careful in checking my ATM and Token Ring information, as those are not my strongest areas.  Thanks.

(cn)Chapter 2

(ct)VLANs

# (1)Certification Objectives

- Broadcast Domains

- Network Security

- Frame Filtering vs. Frame Tagging

- Port-Centric VLANs

- Dynamic vs. Static VLANs

- VLAN Trunks

- ISL, 802.10 and LANE

- Token Ring VLANs

The previous chapter introduced the concepts of bridging and switching.  In this chapter we’ll extend those concepts (bridged local area networks) into Virtual Local Area Networks, or VLANs.

We’ll examine in detail different aspects of VLANs.  This includes ways of assigning clients to VLANs, tracking them between switches, standards for VLANs, and interoperating  with traditional backbone networks.

As much as possible, this chapter covers VLANs in the general sense, without being tied to a particular product.  Obviously, if you’re familiar with the Cisco products, the product-specific discussion of VLANs involves Cisco routers and Cisco Catalyst switches.  Those products are covered in later chapters of this book.  By beginning our discussion of VLANs in a general sense, we can examine why Cisco made some of the product decisions that they did, and learn some alternate ways of implementing VLANs.

When we need to use a particular product as an example, it will be a product from the Catalyst family, unless otherwise noted.

You will be tested on both general VLAN concepts, as well as Catalyst specific VLAN information.  Therefore, even if you have some experience with the Catalyst products, you’ll want to read this chapter for general VLAN questions that will appear on the test.

# (1)Broadcast Domains

In chapter 1, you learned how unicasts, multicasts, and broadcasts traverse a bridged network.  You probably also know from previous study that most LAN protocols rely on broadcasts for many functions.

In chapter 1, the LAN consisted of all the nodes bridged and repeated together.  The defining characteristic of a LAN segment is that all the nodes can communicate directly with each other, without having to pass through some layer 3 or higher device such as a router.  In most cases these direct communications are set up by the nodes sending broadcasts for LAN addresses, and then using unicast LAN addresses to perform the communication.

With traditional LANs, if a node is attached to a particular piece of network equipment (a repeater or bridge) then it is on the same LAN as other nodes attached to the same piece of equipment.  We’ll call this piece of equipment “A”.  Likewise, any other layer 1 or 2 devices that are attached to “A” become part of the same LAN, and nodes attached to that device (we’ll call it “B”) are on the same LAN as devices attached to “A”.  See figure 1.

Figure 1 Two nodes connected by bridges and a repeater

Logically speaking, as long as you stay within the specifications, you can add or remove repeaters and bridges, and you still have a LAN.  If we abstract this a bit further, a LAN is really the group of nodes that can send broadcasts to each other.  A group of nodes that can reach each other by broadcast are said to be in the same broadcast domain.  To state the same thing in a different order for emphasis, a broadcast domain is a group of nodes that can send broadcasts to each other.  A broadcast domain is logically equivalent to a LAN segment.  Broadcast domains are commonly connected together by routers, and usually layer 2 broadcast packets do not cross routers.

We’re being very specific and emphatic about what a broadcast domain is because it’s the central concept behind VLANs.  In the interest of oversimplifying (for now) a {q1}VLAN is a broadcast domain.  There are some extra complications for Token Ring which are discussed in the Token Ring section of this chapter.

## (2) VLAN characteristics

Let’s look at a theoretical VLAN implementation to see how VLAN capable switches differ from traditional switches.  We know that a VLAN is roughly equivalent to a broadcast domain, but that’s also the definition of a LAN, or traditional bridged-and-repeated network.  The difference is really very simple: traditional switches contain one broadcast domain per physical box, and VLAN switches can have more than one.

Despite all the hype surrounding VLANs, it’s really as simple as that.  {q2}The main difference between a plain switch and a VLAN switch is the ability to maintain separate broadcast domains in the same box.  The other factor is the ability to maintain separate broadcast domains between boxes, but we’ll get into that later, in the section on trunking.

# (1)Dynamic vs. Static VLANs

It’s worthwhile discussing how a box might be divided into different broadcast domains.  There are two main categories of VLAN assignment: static and dynamic.  Static VLANs are easiest to understand, so we’ll discuss those first.  {q3}Static VLANs are defined by some characteristic of the box they exist in, usually slots, ports or groups of ports.  For example, is a modular chassis that has 8 slots for networking ports, slots 1-4 may be VLAN 1, while slots 5-8 may be VLAN 2.

Dynamic VLANs are usually defined by some characteristic of the node attached to the box.  {q4}This might be the MAC address of the node, what protocol it’s sending, perhaps even some authentication information, such as a name and password.  An example would be a switch deciding which VLAN to place a node in by watching for the node’s MAC address.

You may have noticed that some of the statements in the paragraphs above are somewhat vague.  The reason for this is that the terms VLAN and switch are somewhat artificial; they are vendor-created.  The end result is that those terms, while not without meaning, are not completely agreed upon by all vendors.  The definitions I’ve been using, and will continue to use, throughout this chapter closely match Cisco’s definitions.  I believe they are as close to correct as any other vendor, and furthermore those definitions will serve you best on the test.  You really only need to be aware of the difference in terms when comparing products from multiple vendors.

{q5}Cisco supports both dynamic and static VLANs.  Cisco dynamic VLANs are based on node MAC address.  Cisco static VLANs are port-assigned.  The term port-assigned implies that VLANs can be assigned on an individual port basis, which is more or less true.  The exception is that Cisco used to sell cards for the Catalyst 5000 family that had ports switched in groups of 4 (i.e. ports 1-4 were repeated together, etc..)  For more details on how to assign VLANs in Cisco switches, consult the chapters for the various Cisco Catalyst products later in this book.

It should be noted that VLAN switches were not the first pieces of equipment to do segmentation within the same box or chassis.  There are a chassis-based repeated solutions, some of which included a few bridge segments on the backplane, which would allow card to be placed on a particular bridge segment, or to be isolated from the backplane entirely.  This provides some of the features of a VLAN.  The difference is that most VLAN switches have the ability to do this completely within software, while the repeater chassis may or may not be separable via software.  Another difference is that most VLAN switches care much more granular, usually allowing VLAN assignment on a per-port basis, and allowing for many more VLANs.  The other main difference is trunking, which we’ll get to later in the chapter.

Now that we know (at least mostly) what a VLAN is, why do we care?  We care because VLANs can help us with practical networking problems.  VLANs exist to help with cost and flexibility.  To explain each of these, we’ll need to look at some examples.

## (3) VLANs can help reduce costs

{q6}One of the items that VLANs help with is cost.  Imagine you want to implement a network of 8 LANs, or more specifically, 8 broadcast domains.  There are many reasons why you might want to implement multiple broadcast domains, such as broadcast control, security, resource grouping, etc..  We won’t go into those reasons here.

With traditional networking technologies, that will require at least 8 sets of networking equipment:

Figure 2 A router connecting 8 LANs using traditional equipment

The word “hub” in the figure, and in the following discussion, is meant to be generic.  It may actually be a hub, which is a common term for a large multi-port repeater, or it may be a bridge, or perhaps a full switch.  For our purposes, it represents the whole set of networking equipment required for each LAN.

If you’re fortunate, the group of nodes that make up each LAN are grouped in such a way as to minimize the number of hubs needed.  One example of a way this might be minimized is if the groups are divided by geography.  That is, maybe each group is made of up a floor of a building, or perhaps it’s done by building.

If you’re unfortunate, groups will be “spread out” causing a need for each LAN to be in physically separated locations, requiring more hubs.  Imagine if your LANs are divided by department, and if most departments are spread among multiple buildings.  Potentially, this could require that a hub for each LAN be in each building.

Figure 3 8 LANs in 2 buildings using traditional equipment

Let’s look at the number of hubs this could require.  In the simplest example, there are 8 LANs.  Each LAN represents a floor of an 8-story building, and one hub is enough for floor.  That requires only 8 hubs.

Let’s look at a worse case.  You have 8 LANs, and each LAN is related to a department.  Your campus consists of 5 buildings.  There is someone from each department in each building.  You’ll require at least 40 hubs for this arrangement, possibly more if one hub per department per building isn’t enough to connect all of the nodes for that LAN in that building.

As you can see, the number of hubs can be large depending on the requirements to separate LANs.  We haven’t yet talked about interconnecting these hubs yet either.  This discussion is a little premature, and we’ll go into detail in the trunking sections, but with traditional equipment, and our worst-case example, we need at least 8 connections to each building.

This is where the concept of being able to divide a box into VLANs starts to look really attractive.  As long as we can fit as many nodes in a hub as needed for each location, we can maintain the minimum number of hubs, and are not constrained by how many LANs we want to have.

## (3) VLANs can help increase flexibility

At this point, the fact that VLANs increase flexibility may already be apparent to you.  The main reason is that with VLANs, where a node is located can be independent of which LAN it’s connected to.  If you consider this in conjunction with our example of LANs mapping to department, this could be a big time saver.  When employees change departments, the network administrator can simple change which LAN the employee’s node is attached to.

Another example is with dynamic VLANs.  Again, using the example where LANs are mapped to department, if employees roam or change locations often, the network administrator can implement dynamic VLANs, and wherever the employee connects their node, they will be on the right LAN.

# (1)Network Security

VLANs can be used to help increase network security.  {q7}Networks can be segmented into VLANs so that nodes on different VLANs must cross a router or perhaps even a firewall in order to communicate with each other.  Additionally, some VLAN switches include the ability to control which nodes can attach to a particular port, to prevent someone from bypassing security mechanisms by switching wires with other nodes.

The previous paragraph is the information you need to answer the questions on the test.  It is, however, not necessarily true.  As part of my regular job, I do a fair amount of network security work.  Like many security professionals, I have fairly high standards for security.  So, while you need to know the above information, I consider it to be largely a selling point.

The scenario where a VLAN switch can provide more security is one where you are able to implement separate VLANs, whereas if you weren’t able to implement VLANs, you would be forced to implement one big LAN.  In that situation, you will have improved security somewhat.

This isn’t a book on security, so a fully detailed discussion isn’t appropriate.  However, let’s look at some quick pointers for implementing as secure as possible VLANs, using Cisco Catalyst switches.

The idea is to prevent nodes on different VLANs from communicating directly with each other without going thorough your layer 3 (or higher) device, i.e. your router or firewall.  In order to make the VLANs as secure as possible, you’ll need to consider turning off the following on each of the ports connecting a user node: flooding, trunking, EtherChannel, spanning-tree, dynamic VLANs, and CDP.  You may also consider turning on “security”, which limits which MAC addresses can transmit to a particular port.

The best advice I can give is to tell you to do your own evaluation of a particular VLAN switch and weigh it against your security needs.

# (1)Frame Filtering vs. Frame Tagging

Consider how a VLAN switch keeps track of which frames are destined for which VLANs.  When a frame enters a switch, it has to determine where to send it.  Traditional switches simple examine the destination of the frame, consult their bridge tables, and forward it to the appropriate port.  If the destination is unknown, or the destination is a broadcast, then the frame is flooded to all ports except the one it came from.

Things are a bit more complicated in a VLAN switch.  In addition to the forwarding decisions that need to be made based on destination address as in the previous paragraph, the source of the frame also has to be considered.

There are at least two obvious ways that the source of a frame can be tracked.  The first is to mark what VLAN it came from when it entered the switch.  {q8}This is known as frame tagging, also known as explicit tagging..  The other is to keep a list of MAC addresses for each VLAN (which the switches must do anyway) and make a determination of whether the frame passes or not after the destination has been determined.  This is called frame filtering, also known as implicit tagging.  Theoretically, frame filtering could also be done on other criteria, such as layer 3 information.

The major difference between the two methods is when the VLAN decision is made.  With frame tagging, the decision is made as the frame enters the VLAN.  With frame filtering, the decision is made when the frame exits the VLAN, or when it may potentially exit the VLAN.  By “exit the VLAN” we mean exiting the VLAN switch, and onto a regular LAN segment, even if it’s only a 5 foot cable between the node and the switch.  In this instance, it does not mean crossing between VLANs, as that is usually done via routers.

The advantage to frame tagging is that the frame is tagged immediately, and no further VLAN membership decisions need to be made on that frame.  Tagging is usually done by adding a field to the frame that contains a VLAN number.  The Cisco documentation sometimes also refers to this as VLAN coloring.  The disadvantage to this approach is that most non-VLAN aware equipment will consider such frame to be invalid, since they don’t follow the standard formats.  Also, there are a number of incompatible implementations of frame tagging, so use of frame tagging will often limit the choice of equipment to one vendor.  The IEEE 802.1q standard is intended to address this issue, by defining a standard frame tagging mechanism.

The advantage to frame filtering is that it does not modify the frames, so there is no problem passing the frames across standard networks.  The disadvantage is that all VLAN devices must be capable of making the identical VLAN determination about each frame.  In practice, this means that if filtering is being done by source MAC address, then all VLAN switches must have a table containing that MAC address and which VLAN it belongs to.  The same goes for other methods, such as layer 3 address, though that may be easier if the VLAN is a direct function of the layer 3 address.  For this type of arrangement to be manageable and scalable, this would really require a protocol for VLAN switches to communicate this information amongst themselves.

For the tests, Cisco wants you to be aware of the difference between the two methods.  In reality, frame tagging has more or less “won.”  All the major switch vendors favor tagging in their implementations, the IEEE standard is for tagging, legacy backbones (FDDI) support a flavor of tagging, and Cisco supports two additional flavors of tagging.  The one exception may be ATM LANE, which more closely resembles frame filtering than tagging.

# (1)VLAN Trunks

Until now, we’ve skipped past the topic of trunks.  The previous section’s discussion regarding frame tagging and frame filtering will start to make more intuitive sense when those ideas are used between switches rather than within a switch.

Earlier in the chapter we said that the main characteristics of VLAN switches are the ability to maintain separate VLANs within a box, and also between boxes.  We’ll now discuss the latter.

For VLAN switches, {q9}a trunk is an interconnect between two switches that carries traffic for more than one VLAN.  This is different from a link between two regular bridges because each switch has to identify which VLAN a frame belongs to when it is received.  This adds some complexity, but also gives us a good deal of flexibility.

Consider our example from earlier in the chapter of the VLANs define by department, in a multi-building campus.  Even though we were able to drastically reduce the number of boxes in each building by combining all the LANs in each building into one box, we still had a separate network link for each LAN between buildings.  With trunking, as long as there is sufficient bandwidth, that can be reduced to one link to each building:

Figure 4: Single link for each VLAN now replace by larger, trunk link.

For this type of arrangement, how to handle frames within a VLAN becomes much more important.  A switch handling frames internally has a lot more flexibility in how it makes VLAN decisions, as it typically has all the information it needs to handle VLANs in a variety of ways.  Once a frame leaves the control of the processor of a switch, and onto a piece of dumb wire, VLANs become a bit more complicated.

In order to decide whether to mark a frame or not, a switch has to know what’s on the other end of the wire it’s placing the frame onto.  The easiest case is an end node, such as a PC or a server.  Typically, end nodes don’t know anything about VLANs (though it is possible to run VLAN-aware software on some operating systems.  This is usually done to “multi-home” a server.)  So, in the case of an end-node, the switch simply has to restore the frame to its original state (or close to it, in the case of a translational or source-routing bridge.)

If the frame is placed on a network segment that contains both end nodes as well as switches, then care must be taken when considering whether to use frame tagging, since some stations may consider such packets illegal.  This may still be OK, as some nodes may simply ignore the frames, or report errors if they track network statistics.

If the frame is being sent just to another switch, the sending switch has to know what type of formats the receiving switch can handle.  This is non-trivial, and there are at least 3 ways to handle this: by static configuration, by advertisement, and by negotiation.

## (2) Static trunk configuration

This type of trunk configuration is easiest to understand.  Simply put, each switch on a trunk is programmed to send and receive frames of a particular trunking protocol only.  In this setup, the port is normally dedicated to trunking, and can not be used for end node attachment.  Static configuration is often useful when automatic negotiation is not working properly or not available.  The disadvantage is that static trunk configuration must be maintained manually.

## (2) Trunk capability advertisement

It’s possible for switches to send periodic announcement frames to indicate that they are capable of performing a particular trunking function.  For example, a switch could advertise that it’s capable of performing a particular type of frame tagging VLAN, so therefore it’s legal to send it tagged frames in that format.  A switch could also potentially go beyond that, and advertise which VLANs it’s willing to trunk for.  This type of trunk setup might be useful for a network segment that is mixed end node and trunk.

## (2) Trunk auto-negotiation

Trunks can also be set up by automatically via a negotiation process.  In this scenario, switches will periodically send frames that indicate that they wish to switch to trunking mode.  If the switch at the other end receives these frames, recognizes them (and is capable of the requested function) and configured to automatically trunk, the two switches will put those ports into trunking mode.  This type of automatic negotiation normally depends on the existence of a point-to-point link between two switches, i.e. that’s all that’s on that network segment.  The port attached to this link is dedicated to trunking, much as with a static trunk setup.

## (2) Cisco switch trunk capabilities

Without getting into too many hardware specifics, or exact trunking protocols (which is the next section) let’s briefly cover which trunk setup methods Cisco switches are capable of.

Like most any VLAN switch, {q10}Cisco switches can be programmed with a static trunking setup.  Cisco switches don’t use advertisement to decide when to send trunk frames as described above, but once the trunk is up by some other means, it does use advertisement to indicate which VLANs are available, and maintain information about those VLANs.  This feature is called the Virtual Trunk Protocol (VTP) and is covered later in the book.  Finally, Cisco switches do support auto negotiation.  The default configuration has this feature semi-disabled, so that every connection between switches does not automatically become a trunk, which you probably would not want.  With the default configuration, if one of the two switches is manually configured to trunk, the other will automatically switch to trunking mode.  This means that only one end of a link needs to be configured to get trunking going.

Not every feature mentioned applies to every Cisco switch, especially the smaller ones.  Consult hardware specific documentation in later chapters of this book.

# (1)ISL, 802.10 and LANE

{q11}Let’s look at some trunking protocols in detail.  Cisco’s Catalyst family of switches support three trunking protocols: Inter-Switch Link (ISL), 802.10, and LAN Emulation (LANE.)  Not all trunking protocols are available on every model of Catalyst switch.  For example, LANE, being an ATM protocol, is not available on the lower-end switches that don’t support ATM.  Consult the hardware-specific chapters later in the book for more information on which switches support which trunking protocols.

ISL and 802.10 are frame tagging VLAN protocols.  LANE isn’t quite either frame tagging or frame filtering, as we will see shortly.

## (2) ISL

{q12}ISL is a Cisco-created, proprietary trunking protocol.  Proprietary in this case means that it was not created or approved by an independent standards body.  It does not necessarily mean that you won’t see it in non-Cisco products.  There are a few NIC cards and drivers available that will do ISL to virtually multi-home servers, and there is a free Linux driver for DEC Tulip-based Fast Ethernet cards.  Other switch vendors are starting to support ISL on their switch products as well.

{q13}Cisco implements ISL on Fast Ethernet ports in its routers and Catalyst switch products.  In the current software, ISL can be used to trunk VLAN traffic for Ethernet, FDDI and Token Ring LANs.  When a frame from one of the supported LAN type needs to be placed on an ISL trunk, an additional frame header is placed at the beginning of the frame, called an ISL header.  We’ll look at the frame header in a moment, but for now we’ll state that it part of it is 10 bits that represent the VLAN number.  10 bits allow for a possible 1024 VLANs, numbered 0 through 1023.  A number of these are reserved, so some of the Cisco documentation states that there are 1000 VLANs that can be used.  The earliest versions of the software only allowed for 256 VLANs.  Consult chapter 7 for more information on software versions.

ISL can carry a wide range of frame sizes.  The ISL headers take up 26 bytes, so those 26 bytes are added onto the size of the frame being trunked.  There is also a new CRC added to the end, which is 4 bytes.  The smallest allowed frame of the protocols trunked is from FDDI, which allows frames as small as 17 bytes, up to Token Ring, which allows packets as large as 18,000 bytes.  So, frames may range in size from 47 bytes, to 18,030 bytes, though the protocol allows for smaller and larger.  This process of adding a new header and checksum is known as encapsulation.  ISL encapsulates the entire frame, without doing any sort of fragmentation.

Obviously, ISL frames at the maximum size are well beyond Ethernet’s maximum of 1,518 bytes (not counting the preamble.)   You might be curious as to why ISL only runs over Fast Ethernet.  There’s no technical reasons why ISL couldn’t be done at 10Mb Ethernet, or over FDDI, or Token Ring.  This choice is more for business reasons than anything else.  Token Ring and 10Mb Ethernet really are just too slow to be useful for trunking.  FDDI is fast enough, but Cisco likely doesn’t want to put a great deal of development time into what is quickly becoming a legacy networking technology.  FDDI is also a worse value, if you use Cisco equipment.  On the Catalyst equipment, a single-port FDDI card costs about the same as a 12-port Fast Ethernet card.  Existing FDDI rings also tend to have legacy equipment connected to them, which would likely not be compatible with the non-standard frames.  Remember that, like Token Ring, FDDI is a logical ring, and each station gets each frame and has to pass it.  Should a station decide a frame is illegal, it might drop it.  Similar problems would happen on a Fast Ethernet link if you tried to run ISL over a link with stations on it, so you should use point-to-point links for ISL.  You could theoretically use point-to-point links with FDDI too, but it would be too expensive.

I was given another reason why ISL doesn’t work over 10Mb Ethernet a couple of years ago.  I had an opportunity to meet with the Catalyst product manager prior to the release of the 5500 family.  I asked him why ISL wasn’t going to work over 10Mb Ethernet.  He said that the chipset they use for 10Mb Ethernet was hard-wired for a maximum frame size of 1,518 bytes, and they weren’t going to start over designing a new 10Mb chipset.  The conclusion on can draw from this, and other evidence, is that the Fast Ethernet hardware is more flexible in terms of what frame sizes it can accept.  This looks good for forward compatibility though software upgrades.  This would be useful if, for example, the proposal to allow larger frame sizes on Fast Ethernet to boost throughput is passed.

Let’s take a look at the ISL header format.  The following table is the contents of the 26-byte ISL header:

| Field | Bits | Function |
| --- | --- | --- |
| DA | 40 | Destination Address – Cisco uses 01:00:0C:00:00. Note that the first 3 bytes are Cisco’s manufacturer ID for MAC addresses, with the multicast bit on. |
| TYPE | 4 | A 4-bit field to indicate the type of frame being carried. |
| USER | 4 | User defined field. May be used to define priority. Note that the DA, TYPE, and USER fields total 48 bits, which would be a normal MAC address size. |
| SA | 48 | Source address – MAC addresses of the switch port sending the frame. |
| LEN | 16 | Length (in bytes) of the frame, minus parts of the header (18 bytes worth.) |
| (fixed) | 24 | This field is fixed, and always equals 0xAAAA03. |
| HSA | 24 | This is equal to the first three bytes of the SA field. Currently must be 0x00000C, which is Cisco’s manufacturer ID. |
| VLAN | 15 | 15 bits to identify the VLAN number. |
| BPDU | 1 | A 1-bit flag to indicate if the encapsulated packet is a BPDU packet. |
| INDX | 16 | Cisco’s documentation says this is the port index of the device that sent the frame. This implies that it’s set to the number that the sending switch uses to identify the sending port internally. Cisco’s documentation says the receiver can safely ignore it, and that it’s used for diagnostic purposes only. |
| RES | 16 | All 0’s for Ethernet. May contain AC and FC fields if carrying a Token Ring frame, or FC if a FDDI frame. |

A few interesting things to note: Cisco has encoded part of the information into what would otherwise appear to be a multicast destination address. The LEN field would seem to allow 64K frames.  Of particular interest is the VLAN field, which is 15 bits long, not 10 as implied.  Cisco is currently using only 10 bits, which allows for 1024 VLANs.  This field would seem to imply that it’s possible to define up to 32,768 VLANs if all 15 bits were used in future versions of the software.

As you can see, ISL is pretty simple as far as networking protocols go.  It just takes the existing frame, throws a (relatively) simple header on the front, and a checksum at them end, and sends it along.  In order to create the simple trunking protocol, the tradeoffs that Cisco makes are that it breaks the Ethernet standard, and requires you to dedicate ports to that purpose.  Overall, it’s a very reasonable tradeoff if you’re using all Cisco equipment, or another vendor who has decided to adopt Cisco’s protocol.

For more information of the ISL frame formats, see:

http://www.cisco.com/warp/public/741/4.html

Note: There is a different ISL header that has been added for Token Ring, and that one is discussed below in the section “Token Ring VLANs”.

## (2) 802.10

Now we’ll examine another trunking protocol, which is less simple, but more interoperable and works across a wider variety of links.  Conceptually, it’s similar to ISL, in that there is an encapsulation of sorts, and a VLAN identifier is part of the new header, but the details are more involved.  Cisco takes advantage of an exiting IEEE standard, 802.10, which was designed to allow “multiplexing” of different LANs across a shared backbone.  In other words, a trunking protocol.  The 802.10 standard includes a 4-byte Security Association Identifier (SAID) which is used to identify when LAN the traffic belongs to.  This allows for over {q14}4 billion different VLANs.  In practice, Cisco software maps the SAID to one of the 1024 VLAN numbers, so the effective limit is still 1024.

Exam Watch: Cisco is sometimes fond of testing numbers in their tests.  The previous paragraph includes a couple that might show up on the exam.

Though Cisco makes it fairly easy to configure it for use, the details underlying 802.10 VLANs are somewhat involved.  The 802.10 “encapsulation” is based on the 802.2 standard, which includes a distinct LLC.

Quick review: The IEEE has divided the OSI layer 2 into two pieces, the Media Access Control (MAC) layer, and the Logical Link Control (LLC) layer.  The MAC portion is responsible for things like preamble bits, collision handling, etc..  The LLC layer is responsible for frame formats and providing a MAC-independent method of accessing layer 2.  The IEEE defines two LLC types, 1 and 2, which are connectionless and connection-oriented, respectively.  LLC type 1 (which is used in 802.10) includes Service Access Points (SAPs) which identify what the function of the frame is.  SAPs are akin to Ethertypes, or port numbers if you’re familiar with TCP/IP transport protocols.

802.10 uses a fixed Link Service Access Point (LSAP) of 0x0A0A03.  A LSAP is a combination of the three bytes that make up the Destination SAP (DSAP), Source SAP (SSAP) and control byte.  This identifies the frame as an 802.10 frame, and tells other stations what additional fields to expect in that frame.

Let’s look at an example.  To keep things simple, let’s start with a generic 802.3 packet:

Figure 5: Simple representation of an 802.3 packet before trunking.

This is how the frame appears before it is placed on the trunk, viewed as a simple block format.  This is what the frame looks like on the trunk:

Figure 6: 802.3 packet while on a trunk link using 802.10 trunking.

Let’s examine the new fields:

| Field | Bits | Function |
| --- | --- | --- |
| Length | 16 | This existed before, but now it’s been increased by 16 bytes, because we’ve added 16 bytes of additional header. |
| LSAP | 24 | This is the LSAP field, fixed at 0x0A0A03 to identify it as a 802.10 frame. |
| SAID | 32 | The SAID field, which holds the VLAN number. |
| Station ID | 64 | Contains the 6 bytes for the source address of the original frame. |
| Frag | 16 | Two bytes to handle fragmentation information. Not supported by Cisco, so they are set to 0. |
| Original Header | ? | This is the original 802.2 LLC header. |

Naturally, the process is reversed when the frame leaves the trunk.  Cisco provides a document with several good example of this process for different original frame types here:

http://www.cisco.com/warp/public/741/3.html

Cisco’s 802.10 VLAN implementation is slightly different from the official 802.10 spec, so some consider Cisco’s implementation to be another proprietary extension.  I’ve seen a presentation from a competing vendor that went so far as to call Cisco’s use of 802.10 “illegal” i.e. it violates the standard.  That’s probably a bit strong of a statement, but keep this in mind if you plan to use 802.10 to try to mix switch vendors.

## (2) LANE

Asynchronous Transfer Mode (ATM) was designed to be a network protocol for many purposes, including acting as a trunk for both LAN and WAN connections.  Not just a data trunk either, since it’s just as capable at trunking voice and video if you have the right ATM equipment.  For now, out interest is in data networking, and specifically in using ATM as a VLAN trunking mechanism.

ATM is covered in detail in chapter 11, and ATM hardware specifics can be found in the various hardware chapters of this book.  Still, you may be reading this book sequentially, and this may be the first time you’ve encountered ATM.  If you’ve never heard of ATM before, you might want to consider reading the ATM chapter first, before this short section on ATM trunking.  For those who are somewhat familiar with ATM, we’ll be doing a short review.  If you’re very familiar with ATM, and have worked with it before, then you probably already know that ATM trunking is just a side effect of how LANE works, and was designed in from the beginning.

For the purposes of discussing LAN Emulation (LANE), ATM is used as a backbone trunk designed to carry large amounts of traffic.  It’s primarily of interest if you need to carry mixed traffic types, such as video and voice in addition to data, or if you need a lot of bandwidth.  ATM comes in various speeds, including OC3 (155Mb) and OC-12 (622Mb.)  These speeds existed before Fast Ethernet and Gigabit Ethernet products were available, so some early adopters who needed a lot of bandwidth may have deployed ATM backbones some time ago.

Exam Watch: There are a whole raft of new acronyms that come with ATM, and you will need to memorize what they stand for and what functions they represent for the test.  We’ll be going over them fairly quickly here, so be sure to read chapter 11 carefully and work on learning the new terms.

Originally when ATM was created, the assumption seemed to be that it would replace all legacy LAN and WAN links, and everyone would have an ATM NIC in their desktop computer, servers, etc..  That didn’t happen, and it doesn’t look like it ever will happen.  Still, ATM has a lot of nice characteristics and many people wanted to use it for specific functions, such as a backbone trunk.  So, the standards bodies behind ATM created a new set of standards called LANE, which allows traditional LANs to use ATM as a backbone trunk.

Let’s jump into the new terms.  The device that does most of the work in a LANE environment is called a LAN Emulation Client (LEC).  A LEC is the software running on a device that has both traditional networking ports (Ethernet, Token Ring, etc..) as well as at least one ATM port.  Be clear on this point: The LEC is NOT the desktop computer attached to the switch, it is the switch itself, or rather the VLAN function of the switch.  In the Cisco world, a Catalyst (or possible LightStream) switch with ATM hardware is the device that has a LEC.  Each separate VLAN which is trunked over ATM has a LEC.  If there are 5 VLANs define, then that switch has 5 LECs.  {q15}Each LEC is associated to with one VLAN.

The term LEC implies that there is a server.  Indeed, there is a LAN Emulation server (LES).  The basic job of the LES is to provide LANE information to the LEC.  He types of services that the LES provides to LECs includes MAC address mappings, MAC registration services, and addresses of other LANE components.  When a LEC adds a new Ethernet station to a VLAN, it registers that stations MAC address with the LES.  We’ll call this station A. When a new station, let’s call it station B, wants to communicate with station A, the LEC that station B is attached to queries the LES for the ATM address of the LEC that has station A.  Station B’s LEC then creates an ATM connection to the LEC that has station A, and forwards the frame.  {q16}Cisco’s implementation allows for only one LES per VLAN.

You may have spotted a problem with the scenario just mentioned, which is what to do if the desired MAC address hasn’t been registered with the LES, or if a broadcast frame is to be sent.   {q17}To solve this problem, a Broadcast-and-Unknown Server (BUS) is added.  As the name implies, it’s involved in forwarding frames that are broadcast (or multicast) frames or frame for which the location is unknown.  The simplified version of how a BUS is used is that when a LEC queries the LES for the LEC that “has” a broadcast or unknown MAC address, it’s given the ATM address of the BUS instead.  The LEC forward the frame to the BUS.  The BUS then creates a copy of the frame for each LEC in the VLAN, and forwards it to them.  Those LECs are then responsible for sending a copy of the frame to each port in that VLAN.  In the Cisco implementation, the LES and BUS devices are the same, and there is only one per VLAN.  Cisco puts this function on their routers.

The final LANE-related device is the LAN Emulation Configuration Server (LECS.)  When a LEC first joins an emulated LAN, it consults the LECS for certain information, such as the address of the LES for that VLAN.  {q18}This happens just once at startup.  The LECS also contains ATM naming information.  One LECS is required per ATM cloud, which is to say all of your interconnected ATM devices.

That’s how ATM trunking works.  Did you miss it?  Using ATM to trunk VLANs is really just a side effect of how ATM handles LANE.  Since LANE is able to deal with more than one emulated LAN across an ATM switch fabric, the trunking capability is automatically there.  The various LANE components take care of checking which ports and stations are allowed on which VLANs.  There’s not a lot to say about trunking on ATM as a separate function, because it’s not separate.  If you develop an understanding of LANE, you will see how the “trunking” portion works.

## (2) Other Trunking Technologies

Though it’s not on the tests yet, it’s worth mentioning the IEEE 802.1q standard.  802.1q is the first official VLAN standard.  Official being defined as meaning written by a recognized independent standards body.  If the implementation is done properly, this is the VLAN standard you should expect to work between all switch vendors who support it.

802.1q has a few options that aren’t available in ISL or 802.10, for example, the IEEE spec claims that tagged frames will work over shared 802 networks as well as point-to-point.  The 802.1q is also a more ambitious standard in that it covers more possible configuration options.  So, a direct comparison isn’t really possible.  If you’re curious, the 802.1q tagged frames (without additional options) resemble 802.10 tagged frames.

Cisco supports 802.1q in the latest available IOS version (12.0T) as of this writing:

http://www.cisco.com/univercd/cc/td/doc/product/software/ios120/120newft/120t/120t1/8021q.htm

Cisco added support for 802.1q in version 4.1 of the Catalyst software:

http://www.cisco.com/univercd/cc/td/doc/product/lan/cat5000/rel_4_1/config/vlans.htm#xtocid1309640

A draft IEEE copy of the spec is available in .PDF format (you’ll require Adobe Acrobat to read it.)  This looks like it may be the final spec, but it’s still in the approval process:

http://grouper.ieee.org/groups/802/1/vlan.html

This page tracks 802.1 news:

http://grouper.ieee.org/groups/802/1/news.html

On the Job: There’s one last “trunking” option I want to mention, and that is  not to trunk.  A set of examples was given at the beginning of the chapter describing the advantages of trunking.  The main reason is to save resources, specifically, ports and cabling.  At the company I work at, we’ve been using Catalyst switches for some time, since just before the Catalyst 5500 family was released.  Basically, this means that we started with the 2.x software.  We’re very fortunate that in our headquarters campus, which consists of several buildings, we own our own underground fiber.  Into each building, we’ve got at least 24 strands of fiber, and in most cases, we’ve got much more than that.

In order to keep things as simple as possible, and make our cutovers from the older equipment easy, we kept one link per VLAN per switch in place.  This means that we were essentially using spanning tree instead of a trunking protocol.  About the same time, our Cisco sales engineer had another customer who was trying to use just about every feature in the Catalyst.  They had a large number of problem and kept the SE quite busy.  We had no such problems.  The moral is that, given sufficient resources, you don’t have to use new features just because they’re there.  That’s an especially good idea if the software isn’t very mature.

# (1)Token Ring VLANs

Why a separate section on Token Ring?  As discussed in Chapter 1, Token Ring bridging is a little different than Ethernet bridging.  The main difference is that Token Ring can use Source-Route Bridging (SRB) as well as transparent bridging, while Ethernet uses strictly transparent bridging.  This makes Token Ring considerably more complicated than Ethernet, but also gives it some extra capabilities, and Cisco wants to maintain those capabilities in their Token Ring implementation. Because of the SRB capability, Token Ring stations can be aware of the logical structure of any interconnected rings.  Token Ring includes the concept of a local-ring only broadcast, and an all-rings broadcast, which is under control of the station sending the broadcast.  A number of protocols implemented on top of Token Ring use that capability.

This type of broadcast arrangement is different from Ethernet.  An Ethernet LAN has one broadcast domain, and indeed the LAN is defined by that broadcast domain.  On Token Ring, there is a possibility for more than one broadcast domain, but still be only one LAN (no router needed to cross rings.)  This allows for a certain amount of hierarchy in the broadcast structure.

In traditional Token Ring, all stations on the same MAU (also sometimes called a concentrator) or interconnected MAUs are on the same ring, and have the same ring number.  If there is more than one of the rings, the may be connected by a bridge or a router.  If connected by a bridge, each ring has a different number.

Cisco Catalyst switches have the capability to define a ring number on a per-port basis for Token Ring.  You can define different ring numbers, and have the Catalyst act as a SRB, or you can define ports to have the same ring number, and have it act like a logical MAU.

When a Catalyst is acting like a logical MAU, and forming a (single) logical ring, it’s said to be performing a Token Ring Concentrator Relay Function (TrCRF).  When the Catalyst is configured with ports in different logical rings, it’s said to be performing a Token Ring Bridge Relay Function (TrBRF).  You can, of course, mix the two functions within a single switch.

Exam Watch: You will most likely see questions on the test involving Token Ring bridging functions.  You will likely only be given the abbreviations (TrCRF, TrBRF) and be expected to answer based on their meaning.  Be sure that you know what they stand for and how they interrelate.  If you are clear on what a concentrator is and what a bridge is you should be able to distinguish the two, but you’re stuck having to memorize what the acronyms stand for.

TrCRFs (with a couple of exceptions discussed below) can only exist within a single switch.  For this purpose, a Catalyst 3900 stack is considered a single switch.  Each TrCRF can have a single TrBRF associated with it.  Stations on different rings (TrCRFs) can communicate with each other at layer 2 via the TrBRF, assuming the TrCRFs share the same parent TrBRF.

Here’s the exceptions.  {q19}There is a default TrCRF that exists in all Token Ring Catalyst switches as VLAN 1003.  It’s called, plainly enough, the default TrCRF.  It’s TrBRF is called the default TrBRF, which is VLAN 1005.  To take advantage of the default TrCRF, the switches must be connected via ISL.  The default TrBRF can only have the default TrCRF associated with it.  The default TrBRF doesn’t perform any bridging functions; it doesn’t need to, as it only has the one TrCRF.

The other exception is the backup TrCRF.  It’s possible to define a single port on multiple switches with the same TrCRF number, if that TrCRF is marked as backup.  You can have one backup TrCRF define per TrBRF shared between switches.  The purpose of the backup TrCRF is to provide another network path if the ISL link between switches, which is providing a path for the TrBRF goes down.  Normally all but one of the ports in a backup TrCRF is down.  When the ISL link goes down, those ports become active, thereby providing an interconnecting ring for stations to communicate across.

## (2) DRiP

{q20}Since Cisco does not allow the use of the same TrCRF in different switches (except for the exceptions noted) they have created the Duplicate Ring Protocol (DRiP).  DRiP is used by switches to inform each other about which TrCRFs they have active.  This information can used to prevent the definition of duplicate TrCRFs on different switches.  DRiP is also used for the following:

- All-Routes Explorer (ARE) filtering – If the switch knows which TrCRFs are attached to a TrBRF, it can correctly drop AREs that have already been to an attached ring.

- Detection and Shutdown of duplicate TrCRFs – If two switches determine that they have the same TrCRF in use, the switch with the higher MAC address will shut down it’s ports in that TrCRF.

- ISL link failure detection – This is related to the backup TrCRF mentioned previously.  DRiP will advertise when ISL links go up and down, so backup links can be activated and de-activated.

Be prepared to answer DRiP questions at about the level shown in the bullet list above.

Cisco provides an excellent set of documents regarding their Token Ring Switching products.  I highly recommend them for further study, particularly the one titled “Token Ring VLANs and Related Protocols”:

http://www.cisco.com/univercd/cc/td/doc/product/lan/trsrb/

# (1)Certification Summary

VLANs expand on the concepts of bridging and switching by allowing the creating of multiple, logical LANs within a single switch chassis.  Further, this concept is extended to trunk links, which allow those logical groups to be kept separate while passing between switches.

There are two main types of trunking mechanisms, frame tagging and frame filtering, also called explicit tagging and implicit tagging.  The use of frame tags is also sometimes called “coloring” a frame.  Frame tagging modifies frames as they enter a switch to somehow mark which VLAN they belong to, to make it easy for other switches to make forwarding decisions.  The modification is removed when the frame leaves the last switch on it’s way to the original intended recipient.  Frame filtering makes forwarding decisions based on some information contained within the frame, without modifying it.  The information filtered on may include MAC address, layer 3 address, or some other information that can be inferred from each frame.

Cisco provides several trunking protocols in their switch and router products.  These include ISL, 802.10, LANE, and in newer versions of the software, 802.1q.  With the exception of LANE, these are all frame tagging trunking protocols.  LANE works a little bit differently, and is closer to frame filtering than frame tagging.  LANE is able to trunk VLAN connections because it inherently includes the concept of multiple LANs being emulated across an ATM switch fabric.

ATM LANE includes a number of pieces that are required for LANE to function.  These include the LAN Emulation Server (LES), LAN Emulation Client (LEC), Broadcast-and-Unknown Server (BUS), and the LAN Emulation Configuration Servers (LECS).  Each of these pieces have a critical function in determining VLAN membership or delivering frames to the intended recipients.

Token Ring VLANs are a bit more complicated than Ethernet VLANs, because Token Ring includes more bridging options.  Cisco divides the two types of Token Ring broadcast domains into Token Ring Concentrator Relay Function (TrCRF) and Token Ring Bridge Relay Function (TrBRF).  With a couple of exceptions, TrCRFs can only live within a single switch.  These are joined, possibly across switches, by TrBRFs.  Cisco also provides the DRiP to help prevent duplicate TrCRFs between switches, and also to help enable some redundancy features.

## (2) Q & A

| How do I decide which trunking protocol(s) to use? | This will depend on your needs and your infrastructure. If you need to interoperate with another switch vendor, you’ll have to find a common protocol. If you have an ATM backbone, you’ll have to use LANE. If you have a FDDI backbone, you’ll want to use 802.10. If you’re building from scratch using all Cisco gear and Fast Ethernet links, ISL is pretty easy to use. |
| --- | --- |
| How do I start using LANE? | Most of the LANE components mentioned here really just live in a Cisco router. Obtain a Cisco router with ATM interface, and one or more Catalyst switched with ATM. Read up on your router ATM commands (most of the work is on the router, not the switches.) |
| Which is better, frame tagging or frame filtering? | It’s a bit of a moot point, unless you’re using LANE. It seems pretty clear that frame tagging is the preferred method. |
| What’s the best way to get started setting up a VLAN infrastructure? | You’ll want to check out Cisco’s VTP protocol, and design in use of that from the beginning. If you’re using Fast Ethernet for trunks, try to build a short tree, without too many three-way links. If you decide to use some of the advanced redundancy features, you’ll be much happier with that type of layout. |
| Any last-minute test tips? | Memorize the acronyms, and be clear on what each one means. Be especially aware of how many of something can be used… i.e. one TrBRF per TrCRF. |

# (1)Two-Minute Drill

We will do the Two-Minute Drill section at Syngress. amw

# (1)Self Test

- A VLAN is roughly equivalent to:

  - A collision domain

  - A broadcast domain

  - A VTP domain

  - A DNS domain

B. Out of the choices given, a broadcast domain is most correct.  This is less correct with Token Ring, but it’s still the best answer in the group for that, too.

- What is the difference between a regular switch and a VLAN switch?

  - VLAN switches can do SNMP

  - VLAN switches have RMON

  - VLAN switches can do multiple layer 3 protocols

  - VLAN switches can maintain separate broadcast domains
D. The main new feature of VLAN switches is that they can have separate broadcast domains in the same chassis. Most regular switches can do all the features mentioned in the other answers.

- Static VLAN assignment is usually based on?

  - Port

  - MAC address

  - IP address

  - VLAN ID

A. Most static VLANs are port-assigned.  B&C are dynamic VLAN assignment criteria.  D exists after assignment has already happened.

- Which of the following are potential dynamic VLAN criteria (pick 2)?

  - MAC address

  - Slot number

  - Port number

  - IP address

A&D. Dynamic VLANs are usually based on some characteristic present in each frame.  B&C don’t usually change the frame in any way.

- Cisco supports which types of VLANs in their switches?

  - Static

  - Dynamic

  - Both

  - Neither

C. Both static and dynamic.

- VLANs can help with which of the following?

  - Decreased costs

  - Reducing latency

  - Increased flexibility

  - Consolidate WAN links

A&C. Two characteristics specifically mentioned in this chapter were decreased costs (over an equivalent number of traditional switches to do the same number of LANs) and increased flexibility.  VLANs don’t improve latency any over regular switches or repeated networks.  WAN links are still primarily the domain of routers, not switches.

- How do VLANs increase security?

  - By bridging between segments

  - By providing RMON capabilities

  - By isolating traffic to specific broadcast domains

  - By providing a firewall feature
C. On the real test, look for terms like isolate and segment for VLAN security questions.  Answer A provides no extra security.  Answer C can work against security, in the wrong hands, and answer D doesn’t apply.. VLAN switches don’t have a proper firewall feature, that’s usually implemented with a separate device.

- Frame tagging is also known as (pick 2):

  - Frame filtering

  - Trunking

  - Explicit tagging

  - Coloring
C&D. Frame tagging is also known as explicit tagging or coloring.

- What is a trunk?

  - A VLAN tagging mechanism

  - A way of passing more than one VLAN over a link

  - A SRB to link TrCRFs

  - A way to bridge between VLANs
B. A trunk is a link designed to carry traffic for multiple VLANs.

- Cisco switches support which kinds of trunk setup (pick 2)?

  - Static

  - Advertisement

  - Auto-negotiation

  - ISL
A&C. Cisco supports static trunk setup, as well as auto-negotiation.  Cisco doesn’t really support trunking based strictly on advertised capabilities, though VTP does part of that function. ISL is a trunk protocol, not a way to establish a trunk.  If that wasn’t clear, you would have had to infer that from the fact that you needed two answers, and ISL didn’t fit with any of the others.  You may need to make similar inferences on the real test.

- Which trunking protocols does Cisco support (select all correct answers)?

  - ISL

  - LANE

  - 802.10

  - STP
A,B&C. ISL, LANE, and 802.10 are correct.  I don’t know what STP is.

- Who created ISL?

  - IEEE

  - ANSI

  - IBM

  - Cisco
D. Cisco created ISL independently.

- ISL works over what types of network links?

  - FDDI

  - Fast Ethernet

  - HDLC

  - ATM

B. So far, ISL only works over Fast Ethernet.

- 802.10 theoretically allows for how many VLANs?

  - 256

  - 1024

  - 32768

  - Over 4,000,000
D. By using 32 bits, there are over 4 million possible VLAN Ids using 802.10.

- Each LEC is associated with how many VLANs?

  - 1

  - 2

  - 4

  - 8

A. Each LEC is associated with only 1 VLAN.  If another VLAN is define in an ATM switch, a new LEC is created to go with it.

- How many LES’ does Cisco support per VLAN in their implementation?

  - 1

  - 2

  - 3

  - 4
A. Cisco only supports 1 LES per VLAN.

- Which LANE component handles broadcasts?

  - LEC

  - LES

  - BUS

  - LECS
C. The Broadcast-and-Unknown Server(BUS) handles broadcasts in LANE.

- How often does a LEC consult with the LECS?

  - Once

  - Each time a broadcast occurs

  - Every 30 seconds

  - Never

A. A LEC consults with the LECS once at startup to obtain configuration information.

- What is the VLAN number for the default TrCRF?

  - 1

  - 1003

  - 1005

  - 1023

B. The correct answer is 1003.

- What is one of the primary functions of the DRiP protocol?

  - To provide SRB for Token Ring LANs

  - LANE configuration

  - To prevent duplicate TrCRFs

  - Packet encapsulation

C. Of the choices given, C is the best answer.